BASIC LINE W-3 Kids – Hot buffet 3 x GN 1/1 with low module height for children's catering

Dimensions:

Width:						1255 mm

Depth:						690 mm

Depth 1:					775 mm

(= with folded-down tray slide on customer side)

Depth 2:					990 mm

(= with folded-up tray slide on customer side)

Height of top surface:			750 mm

Height, including bridge attachment:	1155 mm

The final module equipment depends on the selected configuration.

Basic module:

Mobile module, based on a sturdy, self-supporting sheet metal structure, installed ready to plug in, with approx. 2 m connection cable and plug.

Top surface:

Three deep-drawn bain-marie wells with individual regulation and switching and a stamped fill-level marking are seamlessly welded into the 40 mm high, micro-polished stainless-steel top surface with smooth bevelling on all sides. The bain-marie wells are designed to hold 1/1-GN-200 GN containers or their subdivisions. The wells can be indirectly heated wet or dry, and the power consumption per well is 
0.7 kW. Each well is equipped with a ¾" drain and all wells are connected to a common drain pipe with a central safety drain valve. The safety drain valve is designed in two sections to prevent accidental opening.

The on/off switches and thermostats for the individually switchable bain-marie wells in the heat bridge are installed in the angled well trim panel (switch panel cover) on the operator side to protect them against impact.

Underframe:

The side faces and the trim panel in front of the well section on the customer and operator sides are made of powder-coated thin sheet electrolytically galvanised on both sides. The standard colour of the basic buffet below the stainless-steel top surface:

Traffic grey B (RAL 7043)

The following colours are available for the underframe as an alternative: See under Accessories/B.PRO colour options. *

The castors are mounted on the lower surface of the side faces: 2 twin steering castors with brakes on the operator side, 2 twin steering castors on the customer side, castor diameter: 75 mm.

Accessories/options:

- Body colour: Side faces & panels, powder-coated in B.PRO colours. *

- Front panelling on customer side, between side faces, easy to remove and mount,

- made of thin sheet, electrolytically galvanised on both sides, powder-coated in B.PRO BASIC LINE colours. *

- or particle board, coated with Resopal laminated sheet

- Operator-side front panelling, featured as hinged doors between the side faces.

- Made of thin sheet, electrolytically galvanised on both sides, powder-coated in B.PRO BASIC LINE colours * or faced with Resopal laminated sheet.

- B.PRO colours: *

merlot red 19-1531 TPG

sea blue 19-4234 TPG

petrol green 18-5112 TPG

candy red 17-1562 TPG

neomint 15-5718 TPG

signal white, RAL 9003

stone grey, RAL 7030

traffic grey B, RAL 7043

graphite black, RAL 9011

broom yellow, RAL 1032

lime, Pantone 382 C

apple green, Pantone 370 C

- Resopal laminated sheet in over 180 "Colours" or "Woods" Resopal decors

- Standard bridge attachment with sneeze guard.

The smooth-surfaced, fully panelled bridge attachment of micro-polished stainless-steel sheet is mounted in the centre of the top surface. Stainless-steel brackets for fastening the sneeze guard glass are mounted on the left and right of the bridge attachment. The brackets are U-shaped to protect the glass against damage on the short sides. Made of toughened safety glass, the sneeze guard is mounted, so it slopes towards the customer side and provides a pass-through opening. Pass-through height: 280 mm.

- Highline bridge attachment with toughened safety glass sneeze guard is attached, so that it is horizontal to the top surface on the customer side and provides a pass-through opening. Pass-through height: 395 mm.

- Operator-side sneeze guard made of toughened safety glass.

Pass-through height standard attachment: 
280 mm

Pass-through height Highline attachment: 
395 mm

- Customer-side sneeze guard made of toughened safety glass, without pass-through, down to the top surface.

- Three individually switchable ceramic radiators with a power consumption of 200 W each. Fitted into the bridge attachment. The ceramic radiators are protected against being touched directly with a close-meshed stainless-steel grid plate.

- LED lighting with 3 LED spotlights (or 4 LED spotlights when radiant heater/LED combination) are installed in the bridge attachment. Spotlights can be switched off and on separately from the radiant heaters.

- Tray slide made of stainless-steel round tube (tube 25 mm in diameter) on customer side and/or operator side, fold-down. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Tray slide made of stainless-steel sheet on customer side and/or operator side, fold-down. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Tray slide made of Multiplex panel, faced with Resopal on customer side and/or operator side, fold-down. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Plate slide made of stainless-steel sheet on customer side and/or operator side, fold-down. Flush with the top surface at height of 750 mm. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Plate slide made of Multiplex panel, faced with Resopal on customer side and/or operator side, fold-down. Flush with the top surface at height of 750mm. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Shelf made of stainless-steel round tube (tube 25 mm in diameter) on short side, right and/or left, fold-down. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Shelf made of stainless-steel sheet on short side, right and/or left, fold-down. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Shelf made of Multiplex panel, faced with Resopal on short side, right and/or left, fold-down. With child-safe lock mechanism. Fixed locking thanks to locking pins.

- Switch panel cover top surface To cover switches and operating elements

- Additional socket outlets. Two additional socket outlets with 230 V Schuko socket each, on operator side installed on inside of right side face. The maximum permitted power input for socket outlets to connect external units depends on the unit equipment.

- Mains connection with 16A CEE plug, 400 V

- Cable routing upwards, block length: 450mm, extended approx. 2m

- Base bottom made of stainless steel

- Module connection set to permanently interconnect two buffets, including profile panel to cover the gap between two modules.

- Stainless-steel castors, 125 mm in diameter, 
4 steering castors, 2 of which have brakes. Total height is increased by 60 mm, underframe height is then 810 mm

- Adjustable feet made of stainless steel (instead of castors)

- Stainless-steel skirting panels on customer side/short side right/left (can only be ordered in conjunction with adjustable feet)

Technical data:

Material:	Stainless steel (AISI 304) micro-polished; combined with powder-coated thin sheet electrolytically galvanised on both sides.

Weight:		Depending on equipment

Capacity:		max. 3 x GN 1/1-200

Temperature range:	+30 °C to +95 °C

Connected load:	230 V/16 A/1N PE/

50–60 Hz

The module's connected load depends on the optional accessories.

Special features:

- Individually heated and controlled bain-marie wells for wet or dry operation

- Separately switchable ceramic radiators over each bain-marie well

- Sneeze guard made of toughened safety glass

- Micro-polished surface made of stainless steel

- Two-section safety drain valve

- Fold-down tray and plate slides made of stainless-steel round tube, stainless-steel sheet and Multiplex with Resopal lining and child-safe lock mechanism.

- Switch panel cover to prevent fiddling around with switches with intent.

- Large selection of colour scheme and customisation options

Make:

Manufacturer:		B.PRO

Model:         		BASIC LINE W-3 Kids

Order No.:		381 897