BASIC LINE WQ-3 - Hot buffet 3 x GN 1/1 Diagonal

Dimensions:

Width:					1935 mm

Depth:					  690 mm

Depth 1:				  775 mm

(= with folded-down tray slide on
customer side)

Depth 2:				  990 mm

(= with folded-up tray slide on 
customer side)

Height of the top surface:		  900 mm

Height incl. bridge attachment:	1305 mm

The final module equipment depends on the selected configuration or the selected equipment variant: Smart, Emotion or Design.

Basic module:

Mobile module, based on a sturdy, self-supporting sheet metal structure, installed ready to plug in, with approx. 2 m long connection cable and plug.

Top surface:

Three deep-drawn bain-marie wells with individual regulation and switching, as well as a stamped fill-level marking, are seamlessly welded into the 40 mm high, micro-polished stainless steel (chrome-nickel steel) top surface with smooth bevelling on all sides. The wells are welded in across the width for ergonomic removal of the food.

The bain-marie wells are designed to hold 1/1-GN-200 containers or their subdivisions. The wells can be indirectly heated wet or dry, and the power consumption per well is 0.7 kW. Each well is equipped with a ¾" drain, and all wells are connected to a common drain pipe with a central safety drain valve. The safety drain valve is designed in two sections to prevent accidental opening.

The on/off switches and thermostats for the individually switchable bain-marie wells are installed in the angled well trim panel (switch panel cover) on the operator side to protect them against jolts/impact.

Underframe:

The side faces and the trim panel in front of the well section on both the customer and operator sides are produced from powder-coated thin sheet that is electrolytically galvanised on both sides. Standard colour of the basic buffet below the stainless-steel top surface:

Traffic grey B (RAL 7043)

The following colours are alternatively available for the underframe: Please refer to Accessories/B.PRO colour options.*

A stainless-steel base bottom, for example to hold B.PROTHERM BPT 420 K / KBUH / KBRUH, is provided as standard.

The castors are mounted on the bottom surface of the side faces: 2 twin steering castors with brakes on the operator side, 2 twin steering castors on the customer side, castor diameter 75 mm. The distance from the floor to the bottom edge of the module is approx. 100 mm.

Standard bridge attachment and sneeze guard:

The smooth-surfaced, fully panelled bridge attachment, produced from micro-polished stainless-steel sheet is fitted in the centre of the top surface.  Stainless-steel brackets for attaching the sneeze guard glass are fitted to the left and right of the bridge attachment. The brackets are U-shaped to protect the glass from damage on the short sides. Produced from toughened safety glass, the sneeze guard is fitted in such a way that it slopes towards the customer side and provides a pass-through opening. Pass-through height: 280 mm.

Accessories/options:

- Body colour: Side faces & panels powder-coated in B.PRO colours.*

- Customer-side front panelling, between the side faces, easy to remove and fit

- produced from thin sheet, electrolytically galvanised on both sides, powder-coated in B.PRO BASIC LINE colours.*

- or produced from particle board, faced with Resopal laminated sheet material

- Operator-side front panelling, designed as hinged doors between the side faces.

- produced from thin sheet, electrolytically galvanised on both sides, powder-coated in the B.PRO BASIC LINE colours * or faced with Resopal laminated sheet material.

- B.PRO colours:*

Merlot red 19-1531 TPG

Sea blue 19-4234 TPG

Petrol green 18-5112 TPG

Candy red 17-1562 TPG

Neomint 15-5718 TPG

Signal white, RAL 9003

Stone grey, RAL 7030

Traffic grey B, RAL 7043

Graphite black, RAL 9011

Broom yellow, RAL 1032

Lime, Pantone 382 C

Apple green, Pantone 370 C

- Resopal laminated sheet material in over 180 "Colours" or "Woods" Resopal decors

- Operator-side sneeze guard produced from toughened safety glass. Optionally available with rounded glass.

Pass-through height with standard attachment: 
280 mm

Pass-through height with Highline attachment: 
	395 mm

- Customer-side sneeze guard produced from toughened safety glass, without pass-through, drawn down to the top surface.

- Fold-down sneeze guard for standard bridge attachment. Simply fold the sneeze guard up and down without the need for tools.

- Three individually switchable ceramic radiators with a power consumption of 200 W each. Fitted in the bridge attachment. The ceramic radiators employ a close-meshed stainless-steel grid plate to prevent direct contact.

- LED lighting with 3 LED spotlights (or 4 LED spotlights with radiant heater/LED combination) are fitted in the bridge attachment. Spotlights can be switched off and on separately from the radiant heaters.

- Tray slide produced from stainless-steel round tube (tube 25 mm in diameter) on customer side and/or operator side, fold-down. At a height of 885 mm or a reduced height of 785 mm for children's catering.

- Tray slide produced from stainless-steel sheet on customer side and/or operator side, fold-down. At a height of 885 mm or a reduced height of 785 mm for children's catering.

- Tray slide produced from Multiplex panel, faced with Resopal on customer side and/or operator side, fold-down. At a height of 885 mm or a reduced height of 785 mm for children's catering.

- Tray/plate slide produced from waterproof particle board, ABS edge banding 2 mm with multiplex printing. Corners rounded with radius. Flush with the top surface at a height of 900 mm. Available in 
12 different Resopal colours:

0300-60 Infinity

9450-60 Teal

D421-60 Midori

0335-60 Orange

D362-60 Nile Green

0104-60 Traffic White

0150-60 Tin

0901-60 Black

D483-60 Yellow

D90-60 North Sea

0617-60 Bergamot

9429-60 Parrot

- Plate slide produced from stainless-steel sheet on customer side and/or operator side, fold-down. Flush with the top surface at a height of 900 mm.

- Plate slide produced from Multiplex panel, faced with Resopal on customer side and/or operator side, fold-down. Flush with the top surface at a height of 900 mm.

- Shelf produced from stainless-steel round tube (tube 25 mm in diameter) on short side, right and/or left, fold-down. At a height of 885 mm or a reduced height of 785 mm for children's catering.

- Shelf produced from stainless-steel sheet on short side, right and/or left, fold-down. Flush with the top surface (900 mm) or at a height of 885 mm or reduced height of 785 mm for children's catering.

- Shelf produced from Multiplex panel, faced with Resopal on short side, right and/or left, fold-down. Flush with the top surface (900 mm) or at a height of 885 mm or reduced height of 785 mm for children's catering.

- Storage shelf produced from waterproof particle board, ABS edge banding 2 mm with multiplex printing. Corners rounded with radius. Flush with the top surface at a height of 900 mm. Available in 
12 different Resopal colours:

0300-60 Infinity

9450-60 Teal

D421-60 Midori

0335-60 Orange

D362-60 Nile Green

0104-60 Traffic White

0150-60 Tin

0901-60 Black

D483-60 Yellow

D90-60 North Sea

0617-60 Bergamot

9429-60 Parrot

- Switch panel cover. To cover switches and operating elements.

- Additional socket outlets, multiple installation variants possible in the underframe and side faces.

- Mains connection with 16A CEE plug, 400 V.

- Upwards cable routing, block length: 450mm, extended approx. 2m

- Module connection set for permanently interconnecting two buffets, including profile panel to cover the gap between the two modules.

- Stainless-steel castors, 125 mm in diameter, 4 steering castors, 2 of which have brakes. Total height is increased by 60 mm, underframe height is then 960 mm. The distance from the floor to the lower edge of the module is then approx. 160 mm.

- Adjustable stainless-steel feet (instead of castors). The distance from the floor to the lower edge of the module is then approx. 94 – 114 mm.

- Stainless-steel skirting panels on customer side/short side right/left (can only be ordered in conjunction with adjustable feet)

Technical data:

Material:	Chrome-nickel-steel 18/10 (AISI 304), micro-polished, combined with powder-coated thin sheet that is electrolytically galvanised on both sides.

Weight:		Approx. 130 kg (without options)

Capacity:		Max. 3 x GN 1/1-200

Temperature range:	+30°C to +95°C

Connected load:	230 V / 16 A / 1N PE /

50-60 Hz

The module's connected load depends on the optional accessories.

Special features:

- Individually heated and controlled bain-marie wells for wet or dry operation

- Transversely arranged wells for easy and ergonomic serving of food for children and teenagers

- Separately switchable ceramic radiators over each bain-marie well

- Toughened safety glass sneeze guard

- Micro-polished stainless-steel surface

- Two-section safety drain valve

- Fold-down tray and plate slides produced from stainless-steel round tube, stainless-steel sheet and Multiplex with Resopal lining.

- Large selection of colour scheme and customisation options

Make:

Manufacturer:		B.PRO

Model:         		BASIC LINE WQ-3

Order no.:		392 820