Gastronorm – Tray with smooth edge, specially annealed – BZG-G 1/1-40

Dimensions

Exterior dimensions:

Length:		530 mm

Width:		325 mm

Depth:		  40 mm

Inside dimensions:

Length:		501 mm

Width:		296 mm

Base radius:		R 16 mm

Corner radius:		R 50 mm

# Design

This specially annealed Gastronorm tray is made of AISI 304 stainless steel and is seamlessly deep-drawn. The edge features a smooth design. Exceptionally large corner and base radii ensure that food is completely used up while also making cleaning easy. The BZG-G 1/1-40 has been specially reinforced to meet the specific requirements for combination steamers. The BZG-G has been annealed to improve stability and rigidity.

Technical data

Material:	AISI 304 stainless steel

Standards:	DIN EN 631

Capacity:	up to 20 mm below edge: 	2.6 litres

up to 3 mm below edge:

4.6 litres

Weight:	1,5 kg

# Special features

- Large corner radii (side wall to side wall)

- Increased base radii (base to side wall)

- Special reinforcement for use in combination steamers

Make

Manufacturer:	B.PRO

Model:	BZG-G 1/1-40

Order No.:		  550 504