Gastronorm – Tray with smooth edge BZG 2/1-20

Dimensions

Exterior dimensions:

Length:		650 mm

Width:		530 mm

Depth:		  20 mm

Inside dimensions:

Length:		 623 mm

Width:			 503 mm

Base radius:		R 16 mm

Corner radius:	R 50 mm

# Design

This Gastronorm tray is made of AISI 304 stainless steel and is seamlessly deep-drawn. The edge features a smooth design. Exceptionally large corner and base radii ensure that food is completely used up while also making cleaning easy. The BZG 2/1-20 has been specially reinforced to meet the specific requirements for combination steamers.

Technical data:

Material:	AISI 304 stainless steel

Standards:	DIN EN 631

Content:	Bowl

Weight:	3,4 kg

# Special features

- Large corner radii (side wall to side wall)

- Increased base radii (base to side wall)

- Special reinforcement for use in combination steamers

Make:

Manufacturer:        	B.PRO

Model:               	BZG 2/1-20

Order No.:         	  550 625