# SW 8x5-3 serving trolley

Dimensions

Length:				900 mm

Width:			600 mm

Height:			950 mm

Inside dimensions:

Height between shelves:		275 mm

Shelves:		 800 x 500 mm

Model

The serving trolley is made of stainless steel, AISI 304. There are three welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the serving trolley’s stability.

A noise-insulating antidrumming mat is positioned beneath the shelves.

The serving trolley is mobile on galvanised steel castors (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Stainless-steel waste container AFB 4 x 2 x 2.5, with mounting hooks, 18.5 litres (order no. 555 509)

- Stainless-steel cutlery holder BGN 1/3-150, with mounting hooks, 110 pieces of cutlery

(order no. 555 510)

- All-round railing, for easy self-assembly (order no. 575 523)

- 3-sided railing, for easy self-assembly (order no. 575 526)

- Shelf reinforcing profiles, increases the max. load per shelf by 10 kg, the load-bearing capacity of the overall SW remains unchanged, for easy self-assembly (order no. 575 530)

Panelling sets:

- Stainless-steel panelling on 3 sides, for hanging on the serving trolley, order no: 375449

- Panelling on 3 sides with front stainless-steel hinged doors for easy mounting on serving trolley, order no.: 375450

- Thin-sheet panelling on 3 sides, electrolytically galvanised on both sides, powder-coated in various colours; for hanging on the serving trolley; order no.: 375453

- Thin-sheet panelling on 3 sides with front hinged doors, electrolytically galvanised on both sides, powder-coated in various colours; for easy mounting on serving trolley, order no.: 375454

Mobile work surface:

- Stainless-steel worktop WB 8 x 5 for placing on top of the serving trolley, 8 x 5 including cut-out for two GN containers 1/3-65; 2x GN 1/3-65 included in scope of delivery

(order no. 574 773)

- 1/1 synthetic cutting board with milled juice groove and drain into the GN containers (order no. 574 776)

- 1/1 cutting board, made of solid beech with milled juice groove and drain into the GN containers (order no. 574 775)

- Stainless-steel hanging frame for GN containers, including bar for paper towel roll (order no. 574 783)

Offers space for up to 3 Gastronorm containers GN 1/6

- Synthetic knife holder, 114x162x20 to place on GN 1/6 (order no. 574 774)

- Sneeze guard SW SG made of acrylic glass (PMMA), for easy mounting on tube clamps (order no. 574 782)

- Synthetic worktop set, comprising: Worktop 
WB 8x5 including 2 x GN 1/3-65, 1/1 synthetic cutting board, hanging frame, including bar for paper towel roll, 1 x GN 1/6-200, 
1 x GN 1/3-200, knife holder

(order no. 574 778)

- Wood worktop set, comprising: Worktop 
WB 8x5, including 2x GN 1/3-65,

1/1 wood cutting board, hanging frame, including bar for paper towel roll, 
1 x GN 1/6-200, 1 x GN 1/3-200, knife holder

(order no. 574 777)

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		16.64 kg

Load-bearing capacity
per trolley:		120 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		3

### Special features

- All-round profile edge on the shelves to reinforce the stability of the serving trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:		B.PRO

Model:                               SW 8x5-3

Order no.                           555 528