# SW 6x4-2 serving trolley

Dimensions

Length:				700 mm

Width:			500 mm

Height:			950 mm

Inside dimensions:

Height between shelves:		585 mm

Shelves:		 600 x 400 mm

Model

The serving trolley is made of stainless steel, 
AISI 304. There are two welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the serving trolley’s stability.

A noise-insulating antidrumming mat is positioned beneath the shelves.

The serving trolley is mobile on corrosion-resistant synthetic castors as per DIN 18867, Part 8 (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Stainless-steel cutlery holder BGN 1/3-150, with mounting hooks, 110 pieces of cutlery

(order no. 555 510)

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		10.06 kg

Load-bearing capacity
per trolley:		120 kg

Load-bearing capacity
per shelf:		 80 kg

Number of shelves:		 2

### Special features

- All-round profile edge on the shelves to reinforce the stability of the serving trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:		B.PRO

Model:                                   SW 6x4-2

Order no.		569 767