# SAW 4 – food serving trolley

Dimensions

Length:					1615 mm

Width:					  714 mm

Height:					  933 mm

Description

Structure

The food serving trolley is made of high-quality, corrosion-resistant stainless steel with a specially ground matte surface finish (“microlised” surface). Deep-drawn heat retaining basins are welded into the trolley top without joints. They can be heated separately either wet or dry and feature separate temperature control. The basins are surrounded by a raised edge. The numbers of the basins are embossed into the trolley top. Each basin has a drain tap on its underside that is protected against inadvertent opening.

A recessed control panel at the short side of the trolley features an on-off switch, a control indicator, and a temperature control. The temperature control allows for continuous temperature setting for each individual basin. On the trolley side opposite the operator side, a helix power supply cord with a dummy plug socket is located.

A CNS safety push-pull handle with side bumper elements of plastic (polyamide) is mounted at the operator side. It also serves as a protection for the control elements.

In the open underframe, a shelf is welded in between four stable rectangular tubes.

The trolley is made mobile by corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm dia.).

Very robust synthetic corner guards (polyamide) at all four corners protect the trolley from being damaged.

### Accessories / options

- foldable shelf mounted at short and/or long sides

- 1/3-to-2/3 folding lids, both usable as shelves

- GN lid support for GN lids size 1/1 or their subdivisions, mounted at short side

- Sneeze guard of ESG safety glass extends down to top surface on customer side or with a pass-through height of 275 mm on customer side

Optionally available with a heating shelf of CNS

(4 ceramic radiators, each 250 W, 230 V, not adjustable, switched together with particular basin)

Length of glass shelf: 1,380mm

Height including attachment: approx. 1,325 mm

- for other castor variations, please refer to our general price list

- Working height (top surface upper edge) configurable between 700 mm and 899 mm (standard: 900 mm)

##### Technical data

Material:		1.4301 / 1.4016

Weight:		56.5 kg

Capacity:		4 heat retaining basins

Heat retaining basins:	for GN 1/1-200 or their subdivisions

temperature adjustable from +30°C to +95°C

Voltage / wattage:	220-240V / 50-60Hz / 2.8kW

### Special features

- protected against splash and jet water (IPX 5)

- top with raised edge

- basins can be heated wet or dry

- basins can be heated and controlled individually

- drain tap protected against inadvertent opening

- suitable for energy-consumption optimisation systems

- complies with DIN 18867, Part 7

Make

Manufacturer:			B.PRO

Model:				SAW 4

B.PRO INMOTION

Order No.:			572 155