# Shelf Trolley RWR-VP 163

Dimensions

Length:				  671 mm

Width:			  832 mm

Height:			1675 mm

Interior dimensions:

Space between support rails:	    115 mm

Design

The shelf trolley consists of CNS 18/10 (AISI 304), Material No. 1.4301. U-profile guide rails are welded between the stable 25 x 25 mm square tubes (curved without a recess). These are supplied with a tip safety as well as push-through protection on both sides. The push-through protections are achieved with foldovers to avoid abrasion while inserting or removal of the trays.

The shelf trolley has a crosswise insertion for EN trays, enabling 2 trays to be inserted one after another.

A 20 x 10 mm square tube welded in elevated at mid-height on both the front and back of the shelf trolley provides additional stability.

The shelf trolley can be moved on stainless synthetic castors pursuant to DIN 18867, Part 8

(4 steering castors, 2 of which have locking brakes, castor diameter 125 mm). Very robust synthetic corner guards (polyamide) at all four trolley corners protect the trolley from being damaged.

Accessories and options

- Dust protection hood made of transparent polyethylene (Order No. 573 018)

- Insulating hood made of polyethylene, aluminized on both sides (Order No. 573 017)

- CNS paneling, 2 or 3-sided

- Transport circlip on both sides

Technical data

Material:	Chrome-nickel-steel 18/10 (AISI 304),

Polyamide (PA)

Material thickness of the

square tube:	1.5 mm

Material thickness of the

support rails:	1.2 mm

Weight:	18 kg

Load-bearing capacity

per trolley:	240 kg

Max. load-bearing

capacity per support pair: 	30 kg

Number of support

pairs:	12

Capacity:	24 EN/ GN trays

### Special features

- U-profile guide rails with tip safety and push-through protection on both sides

Make

Manufacturer:		B.PRO

Model:                               RWR-VP 163

Order No.:		572 923