# SW 10x6-3 BASIC serving trolley

Dimensions

Length:					1100 mm

Width:				  700 mm

Height:				1290 mm

Inside dimensions:

Height between shelves:		  445 mm

Shelves:		   		1000 x 600 mm

Model

SW 10x6 serving trolley in basic design.

The serving trolley is made of stainless steel,
AISI 304. There are three welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the serving trolley’s stability.

A noise-insulating antidrumming mat is positioned beneath the shelves. The spacing between shelves is designed to enable the insertion of GN lids in a GDD top-mounted frame (order no. 574 167). The serving trolley is intended for the correct storage, drying, transport and provision of Gastronorm lids.

The serving trolley is mobile on corrosion-resistant synthetic castors as per DIN 18867, Part 8 
(4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- GDD top-mounted frame made of stainless-steel for holding GN lids (order no. 574 167)

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		21.8 kg

Load-bearing capacity
per trolley:		150 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		3

Capacity:		3 GDD top-mounted frames

### Special features

- All-round profile edge on the shelves to reinforce the stability of the serving trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:		B.PRO

Make:	SW 10x6-3 BASIC GDD

Order no.		574 166