# Serving trolley 8x5-3, including panelling on 3 sides and doors in lime

Dimensions

Length:				900 mm

Width:			600 mm

Height:			950 mm

Inside dimensions:

Height between shelves:		275 mm

Shelves:		 800 x 500 mm

Design

The serving trolley is made of stainless steel, AISI 304. There are three welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all make the serving trolley sturdier.

A noise-insulating antidrumming mat is positioned beneath the shelves.

The serving trolley is mobile on galvanised steel castors (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyamide) wall guards at all four corners protect against damage.

Accessories/options

- Stainless-steel waste container AFB 4 x 2 x 2.5, with mounting hooks, 18.5 litres (Order No.: 555 509)

- Stainless-steel cutlery holder BGN 1/3-150, with mounting hooks device, 110 pieces of cutlery

(Order No.: 555 510)

Panelling set:

The panelling set is made completely of powder-coated thin sheet electrolytically galvanised on both sides. It consists of two side walls, one rear panel and two hinged doors at the front. The panelling sections are folded over and have an edge at the top on the inside with which the panelling can be clamped behind the roller-burnished edge of the upper serving trolley shelf without using tools. With the aid of a second edge on the underside, these are securely fastened on the profile edge of the bottom serving trolley shelf. There are two ergonomic handle cut-outs each at the top of the side walls and the rear panel. These can be used to simply and safely hook in the panelling sections on the serving trolley. The panelling sections are completely removable for cleaning. The hinged doors are fastened on the front on the round pipe clamps of the serving trolley using sturdy hinge parts made of glass-fibre reinforced synthetic. The doors can be opened by 270° and can be locked within the bumper rail. Fixing is carried out with magnets mounted in corresponding cut-outs in the side panelling. The closed doors are locked with an additional magnet, which is mounted at the centre and front on the inside of the top and bottom serving trolley shelves.

Mobile work surface:

- Stainless-steel worktop WB 8 x 5 for placing on top of the serving trolley, 8 x 5 including cut-out for two GN containers 1/3-65; 2x GN 1/3-65 included in scope of delivery

(Order No.: 574 773)

- 1/1 synthetic cutting board with milled juice groove and drain into the GN containers
(Order No.: 574 776)

- 1/1 cutting board, made of solid beech with milled juice groove and drain into the GN containers
(Order No.: 574 775)

- Stainless-steel hanging frame for GN containers, including bar for paper towel roll (Order No.: 574 783)

Holds space for up to 3 Gastronorm containers GN 1/6

- Synthetic knife holder, 114x162x20 to place on GN 1/6 (Order No.: 574 774)

- Sneeze guard SW SG made of acrylic glass (PMMA), for easy mounting on tube clamps (Order No.: 574 782)

- Synthetic worktop set, comprising: Worktop WB 8x5 including 2 x GN 1/3-65, 1/1 synthetic cutting board, hanging frame, including bar for paper towel roll, 1 x GN 1/6-200, 1 x GN 1/3-200, knife holder

(Order No.: 574 778)

- Wood worktop set, comprising: Worktop WB 8x5, including 2x GN 1/3-65,

1/1 wood cutting board, hanging frame, including bar for paper towel roll, 1 x GN 1/6-200, 1 x GN 1/3-200, knife holder

(Order No.: 574 777)

Technical data

Material:		Stainless steel

Polyamide (PA)

Panelling material:		Thin sheet, electrolytically galvanised on both sides, powder-coated

Panelling thickness:		1.25 mm

Weight:		approx. 37 kg

Load-bearing capacity 
per trolley:		120 kg

Load-bearing capacity 
per shelf:		  80 kg

Number of shelves:		    3

### Special features

- All-round profile edge on the shelves to reinforce the serving trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

- Side and rear panel completely removable for cleaning

Make

Manufacturer:		B.PRO

Model:	Serving trolley 8x5-3, including panelling and lime doors

Order No.:		574 815