# CHV 58/ 58 – platform dispenser

Dimensions

Length:					  945 mm

Width:					  700 mm

Height:					  911 mm

Height (with domed lid):			1079 mm

Operating height:				  900 mm

Description

Structure

The platform dispenser is made entirely of CNS 18/10 (AISI 304) with a specially ground matte surface finish (“microlised” surface).

The platform can be used for various types of crockery. The dispenser has a double-walled and insulated body.

Its interior panelling is entirely powder-coated to prevent the crockery from slipping and getting damaged.

The platform is supported by tension springs. By hanging or unhanging individual springs, the serving height can be adjusted according to the weight of the objects stacked.

A recessed control panel at the short side of the dispenser features an on-off switch, a control indicator, a helix cord, a dummy plug socket, and a temperature control. The temperature control allows for continuous setting of the forced-air convection heating. The platform dispenser also has an ECO control range. Within this control range, mid-range plate temperatures can be maintained in the most energy-efficient way.

The transparent domed lid protects the dispenser against heat loss and dirt.

A CNS safety push-pull handle with side bumper elements of plastic (polyamide) is mounted at the operator side.

The dispenser is made mobile by corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm dia.).

Very robust synthetic corner guards (polyethylene) at all four corners protect the dispenser from being damaged.

### Accessories / options

- for other castor variations, please refer to our general price list

Technical data

Material:		CNS 18/10 (AISI 304)

Weight:		85 kg

Capacity:		depends on type of crockery

Interior dims.:		581 x 581 mm

Domed lid:		617 x 617 mm, polycarbonate

Stacking height w/

domed lid:		680 mm

Stacking height w/o

domed lid:		540 mm

Interior:		temperature adjustable from
		+30°C to +110°C

Voltage / wattage:		230V / 50Hz / 1.8kW

Emissions:		The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- protected against splash and jet water (IPX 5)

- ECP control range to heat up the dishes and maintain its temperature of >/= 60°C energy-efficiently.

- entirely powder-coated interior panelling

- adjustable spring tension

- homogenous heat distribution due to forced-air convection heating

- complies with DIN 18665, Part 6

Make

Manufacturer:			B.PRO

Model:				CHV 58/ 58

Order No.:			574 890