# Special Plate Dispenser 2 SHVS 26

Dimensions

Length:						1168 mm

Width:						  548 mm

Operating height:				  970 mm

Height (with CNS cover)			1067 mm

Use

The special plate dispenser is used in the catering industry.

It is used for heating up wax-filled CNS lower sections (heat retaining plates).

Design

## Body

The plate dispenser is made entirely of CNS 18/10 (AISI 304). The surface is micro-polished.

The body, cover and both CNS covers are entirely insulated with a high-quality ceramic fleece, which provides high heat insulation.

##### Cover

Two double-walled insulated CNS covers are easy to operate through a hinge and provide excellent insulation. The CNS lock is opened and closed safely by a CNS clamp. When closed, the cover is securely held by two latching devices to the right and left of the cover. The covers have a circumferential, removable foodstuff-resistant sealing frame.

##### Construction

With the aid of 4 CNS guide poles per stacking well, the two stacking wells can be loaded with wax-filled CNS lower sections with a diameter of 26 cm. The special plate dispenser is equipped with 2 high performance convection-blower heaters, which guarantee quick heating of the wax-filled CNS lower sections. The convection-blower heaters are located on each of the short sides of the unit.

To ensure a consistently even distribution height, the spring tension can be regulated depending on the stacked items by hooking or unhooking the tension springs.

The lower-lying operating panel has an on/off switch, pilot light, helix cable and a dummy plug socket on the short side.

A CNS safety push handle with side synthetic (polyamide) corner guard elements, which protects the switch elements and hands, is located on the operator side.

The dispenser can be moved on stainless synthetic castors pursuant to DIN 18867 (4 steering castors, 2 of which have locking brakes with a diameter of 125 mm).

Stable synthetic (polyamide) corner guards at all four corners protect the unit against damage.

All wearing parts (e.g. castors, corner guards, push handle, locks etc.) can easily be replaced if service is required.

### Accessories and options

- Galvanized-steel castors, 4 steering castors, 2 with brakes

- CNS castors, 4 steering castors, 2 with brakes

### Technical data

Material:	CNS 18/10 (AISI 304)

Weight:	82 kg

Capacity:	98 pieces of wax-filled CNS lower sections (from Hepp)

Maximum loading:	120 kg

Interior dimensions:	dia. 260 mm

Number of stacking wells:	2

Stacking height with

CNS cover:		664 mm

Unit interior:		Temperature approx. +135°C,

not adjustable

Connected load:		220-240V / 50 Hz / 3.0 kW

Emissions:		The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- Protected against splashed and sprayed water (IPX 5)

- Spring tension can be changed

- Especially for wax-filled lower sections

- Suitable for power-optimization systems

- Pursuant to DIN 18665, Part 6

Make

Manufacturer:	B.PRO

Model:		2 SHVS 26

Order No.:		574 902