# UNI-H 58/ 58 – universal dispenser

Dimensions

Length:					  944 mm

Width:					  700 mm

Height					  944 mm

Height (w/ optional domed lid):		1077 mm

Operating height:				  900 mm

Description

Structure

The universal dispenser is made entirely of CNS 18/10 (AISI 304) with a specially ground matte surface finish (“microlised” surface).

It has a double-walled body in a closed and compact design. The interior is lined entirely with a powder-coated panelling to avoid marks on the crockery.

The frame of the platform has been designed so that it allows for stacking a wide variety of different types of crockery with the help of 8 synthetic guide rods that can be variably positioned.

The stacking platform runs smoothly and has ball-bearing roller guides to secure it against tilting and jamming.

The platform is supported by tension springs. By hanging or unhanging individual springs, the serving height can be adjusted according to the weight of the objects stacked.

A recessed control panel at the short side of the dispenser features an on-off switch, a control indicator, a helix cord, a dummy plug socket, and a temperature control. The temperature control allows for continuous setting of the forced-air convection heating. The universal dispenser also has an ECO control range. Within this control range, mid-range plate temperatures can be maintained in the most energy-efficient way.

The transparent domed lid protects the dispenser against heat loss and dirt.

A CNS safety push-pull handle with side bumper elements of plastic (polyamide) is mounted at the operator side.

The dispenser is made mobile by corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm dia.).

Very robust synthetic corner guards (polyamide) at all four corners protect the dispenser from being damaged.

### Accessories / options

- coloured panelling

- for other castor variations, please refer to our general price list

Technical data

Material:		CNS 18/10 (AISI 304)

Weight:		91 kg

Interior dims.:		581 x 581 mm

Capacity:		depends on type of crockery

Domed lid:		617 x 617 mm, polycarbonate

Stacking height w/

domed lid:		680 mm

Stacking height w/o

domed lid:		540 mm

Interior:		temperature adjustable from
		+30°C to +110°C

Voltage / wattage:		220-240V / 50Hz / 1.8kW

Emissions:		The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- protected against splash and jet water (IPX 5)

- ECP control range to heat up the dishes and maintain its temperature of >/= 60°C energy-efficiently

- adjustable spring tension

- variable setting due to 8 guide rods

- suitable for energy-consumption optimisation systems

- complies with DIN 18665, Part 6

Make

Manufacturer:			B.PRO

Model:				UNI-H 58/ 58

Order No.:			574 910