# RWR-E 6x4 shelf trolley

Dimensions

Length:				  686 mm

Width:			  536 mm

Height:			1,515 mm

Inside dimensions:

Spacing between support rails:	  245 mm

Design

The shelf trolley consists of stainless steel, 
AISI 304. Welded-in guide rails are located between the sturdy 25 x 25 mm square tube (curved without tapering). These are equipped with push-through protection on both sides for the lengthwise insertion of Euro containers.

A 20 x 10 mm welded-in square tube on both the front and back provides additional stability.

The shelf trolley is mobile on corrosion-resistant synthetic castors pursuant to DIN 18867, Part 8 (4 steering castors, 2 of which have brakes, castor diameter 125 mm). Synthetic (polyamide) wall guards at all four corners protect against damage.

Technical data

Material:	 	Stainless steel 			Polyamide (PA)

Material thickness 
of the square tube:	1.5 mm

Material thickness 
of the support rails:	1.25 mm

Weight:		14 kg

Load-bearing capacity
per trolley:	    125 kg

Max. load-bearing

capacity per support pair: 	30 kg

Number of support
pairs:		5

Capacity:		5 E2 Euro containers

### Special features

- Guide rails with push-through protection on both sides

Make

Manufacturer:		B.PRO

Model:	RWR-E 6x4

Order no.		575 102