# CE 65/45 platform dispenser

Dimensions

Length:					893 mm

Width:					578 mm

Height:					911 mm

Model

Design

The dispenser housing is made of high-quality, corrosion-resistant stainless steel. The surface is micro-polished.

The platform is used to hold various types of dishes. Depending on stacked items, a range of platforms are available.

The platform dispenser has continuous, powder-coated inner panelling to stop dishes from sliding, thus preventing damage to the body interior.

To ensure a consistently even serving height, the spring tension can be regulated depending on the stacked items by hooking or unhooking the tension springs.

A stainless-steel safety push handle with synthetic (polyamide) side corner guard elements is attached to the operator side.

The dispenser can be moved on its corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm in diameter). Solid synthetic (polyamide) corner guards at all four corners protect against damage.

### Accessories/options

- Push handles as tube clamp made completely of stainless steel

- All-round impact bars made of PE

- All-round impact bars, reinforced (milled)

- Double-walled stainless-steel hooded cover

- Galvanised steel castors, 4 steering castors, 2 of which have brakes, 125 mm dia.

- Stainless-steel castors, 4 steering castors,
2 of which have brakes, 125 mm dia.

Technical data

Material:		Stainless steel

Weight:		approx. 60 kg

Max. load:		220 kg

Capacity:		dependent on type of dishes

Loading:

(Platform for:)		Soup bowls 0.35 l, 104 mm dia., approx. 600 pcs.

Side bowls 122 x 89 mm
		approx. 950 pcs.

Serving height:		900 mm

Inside dimensions:	approx. 656 x 456 mm

Stacking height:		approx. 600 mm

### Special features

- Spring tension can be changed

- As per DIN 18665, Part 9

Make

Manufacturer:			B.PRO

Model:				CE 65/45

Order No.:			575289