# RWR 161 KS shelf trolley

Dimensions

Length:				  663 mm

Width:			  734 mm

Height:			1647 mm

Inside dimensions:

Spacing between support rails:	     73 mm

Model

The shelf trolley consists of stainless steel- U-profile guide rails are welded in between the sturdy 25 x 25 mm square tube (curved without a recess). These are supplied with a tip safety as well as push-through protection on both sides. The shelf trolley has a crosswise insertion for Gastronorm, enabling 2 trays/sheets to be inserted one after another.

A 20 x 10 mm welded-in square tube at mid-height offset on both the front and back of the shelf trolley provides additional stability.

To prevent cold loss, the shelf trolley has a one-piece stainless steel bottom plate.

The shelf trolley is mobile on corrosion-resistant synthetic castors pursuant to DIN 18867, Part 8

(4 steering castors, 2 of which have brakes, castor diameter 125 mm). Synthetic (polyamide) wall guards at all four corners protect against damage.

Accessories

- Dust protection hood made of transparent polyethylene (order no. 568 772)

- Insulating hood made of polyethylene, aluminised on both sides (order no. 568 775)

Technical data

Material:			Stainless steel, 			polyamide (PA)

Material thickness of the square tube:	1.5 mm

Material thickness

Support rails:			1.2 mm

Weight:		25 kg

Load-bearing capacity per trolley:		200 kg

Number of support pairs:		18

Capacity:		18 x GN 2/1 or 36 x GN 1/1

### Special features

- U-profile guide rails with tip safety and push-through protection on both sides

Make

Manufacturer:		B.PRO

Model:                                   RWR 161 KS

Order no.		575 379