# SW 9x6-3 serving trolley

Dimensions

Length:				1000 mm

Width:			  650 mm

Height:			  950 mm

Inside dimensions:

Height between shelves:		  275 mm

Shelves:		   900 x 550 mm

Model

The serving trolley is made of stainless steel, 
AISI 304. There are three welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the serving trolley’s stability.

A noise-insulating antidrumming mat is positioned beneath the shelves.

The serving trolley is mobile on air-identical galvanised steel castors (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Stainless-steel waste container AFB 4 x 2 x 2.5, with mounting hooks, 18.5 litres (order no. 
555 509)

- Stainless-steel cutlery holder BGN 1/3-150, with mounting hooks, 110 pieces of cutlery

(order no. 555 510)

- All-round railing, for easy self-assembly (order no. 575 524)

- 3-sided railing, for easy self-assembly (order 
no. 575 522)

- Shelf reinforcing profiles, increases the max. load per shelf by 10 kg, the load-bearing capacity of the overall SW remains unchanged, for easy self-assembly (order no. 575 531)

Panelling sets:

- Thin-sheet panelling on 3 sides, electrolytically galvanised on both sides, powder-coated in various colours; for hanging on the serving trolley; order no.: 383341

- Thin-sheet panelling on 3 sides with front hinged doors, electrolytically galvanised on both sides, powder-coated in various colours; for easy mounting on serving trolley, order no.: 383460

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		21.04 kg

Load-bearing capacity
per trolley:		120 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		3

### Special features

- All-round profile edge on the shelves to reinforce the stability of the serving trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:	              B.PRO

Model:                             SW 9x6-3

Order no.                         575 501