# Tray transport trolleys

# double-walled model, for passive cooling with eutectic plates

# TTW-PK 24-115 DZE

Dimensions

Length:	 1115	mm

Width:	   754	mm

Height:	 1825	mm

Model

Design

The trolley body is closed on all sides and made completely of stainless steel. The surface is micro-polished. The edged bottom panel is reinforced with longitudinal profiles on the underside. The doors are double-walled and insulated with polystyrene hard foam plates. Additional insulation on the top and unit bottom ensures lower temperature loss in the interior. Interior and exterior body are vertically connected with synthetic profiles and thermally separated.

Side walls and the dividing wall in the compartment are designed for holding trays with seamlessly deep-drawn support ledges with spacing of 115 mm. To prevent accidental tipping when pulling out trays, two seamlessly deep-drawn tip safety mechanisms are provided above the tray supports with a spacing of 7 mm. The support ledges have a width of 11.5 mm for safely holding Euronorm trays (530 x 370 mm). The self-supporting design enables use both inside and outside the building.

There are two holders for eutectic plates on the inside of each of the two doors. The holders consist of two support frames and one condensation-water catch tray. The lower support frame and the condensation-water catch tray are designed as a single component. This ensures that the catch trays are always used. Because the eutectic plates are portioned on the inside of the doors, each compartment is cooled separately. The individual compartments can be opened independently from one another.

The double-walled hinged doors can be opened by 270° and safely locked using a spring-loaded synthetic moulded part on the bottom of the tray transport trolley. The doors locked in the open position are located within the bumper rail to prevent damage. In addition, the doors have an all-round removable silicone seal profile to prevent temperature losses.

The door lock mounted vertically in the middle of the doors is self-closing and designed with integrated transport lock. The sturdy door hinges are welded and protected against impacts, as they are located within the bumper rail.

Use/ergonomics

The tray transport trolley is designed to transport portioned food on trays – especially for early portioning of evening meals.

It is equipped with a sturdy all-round bumper rail, which protects the trolley from damage. Two vertical, ergonomically positioned push handles are located on both short sides. The push handles are located within the bumper rail to prevent the hands from being crushed when rolling through doors and narrow corridors. The push handles are accessible even with the door open.

The tray transport trolley is mobile on 2 fixed castors and 2 steering castors with brakes, a diameter of 160 mm and a galvanised steel housing.

### Special features

- Self-closing door locks with integrated transport lock

- Doors with removable all-round seal profile

- Interior and exterior body are vertically connected with synthetic profiles and thermally separated

- Holders for eutectic plates on the inside of the doors

##### Technical data

Material:	stainless steel, AISI 304

polyethylene (PE)

Insulating material

Doors, walls, top

and unit base:	polystyrene hard foam plates (30 mm)

Weight:	approx. 152 kg

Capacity:	24 Euronorm trays (530 x 370 mm) or

Insert type:	lengthwise insertion

Number of holders

for eutectic plates:	4

Spacing of

support ledges:	115 mm

Spacing of

tip safety / ledges:	7 mm

Number of cabinet

compartments:	2

Number of hinged

doors:	2

Interior design:	hygienic type HS,

in compliance with DIN 18865-9

Accessories/options

- Eutectic plate (-3°C), made of plastic (order no. 575306)

- Eutectic plate (-12°C), made of plastic (order no. 575307)

- Model without tip safety for the use of compact trays (width of support ledges: 14 mm)

- Hygiene type H1, only for double-walled trolley model

- Suitable for washing systems, only in connection with hygiene type H1, door model without door seal, with CNS push handle, all-round milled bumper rail castors suitable for washing systems

- Door model without door seal

- Milled all-round bumper rail made of polyethylene

- Additional milled bumper rail on top

- Additional milled bumper rail on top, all-round

- Additional bumper rail on body, 3-sided

- Transport lock

- Railing on 3 sides, made of stainless steel

- Railing on 4 sides, made of stainless steel

- Draw bar and coupling made of stainless steel, can be locked on the short side

- Antistatic conveyor

- Push handle, CNS model

- Steering castors with central locking brake

- Steering castors with central direction brake

- Castor diameter is 200 mm

- Stainless-steel castors with elastic tyres

- Castor arrangement "D" 4 steering castors with central locking brake and 2 fixed castors on middle of long side

Make:

Manufacturer:	B.PRO

Type:	TTW-PK 24-115 DZE

Order no.:	575 589