# TS-UH1 18-33 plate dispenser

Dimensions

Length:				680 mm

Width:				520 mm

Height: (incl. plate guide)                          1030 mm

Height: (incl. hooded cover)                      1120 mm

Working height:	900 mm

Model

The dispenser housing is made of high-quality, corrosion-resistant stainless steel. The surface is micro-polished. The body is double-walled and insulated.

The plate dispenser is equipped with a square plate tube. The plate tube has a particularly flat stacking platform with jam-proof guides.

Using four Rilsan-coated plate guide rods, it can be adjusted to various plate sizes and plate shapes flexibly and without tools using a hole pattern on the bottom. For example, round plates with a diameter of 18 – 33 cm and rectangular plates up to a maximum of 28 x 28 cm can be stacked in the plate tube.

The plate guide rods project above the top surface of the dispenser. This means that more plates can be securely transported and unstacked and the dishes are still guided securely when the hooded covers have been removed.

The plate tube can be completely removed from the dispenser to clean the dispenser easily and set the spring tension simply and ergonomically.

To ensure a constant, even serving height, the tension springs can be to adjust the spring tension to hold the particular stacked items in question.

This setting can be made quickly and effortlessly because of special cut-outs in the spring retainers.

The plate dispenser is equipped with a powerful convection-blower heater.

The operating elements and the helix cord are recessed in the switch panel on the short side to prevent damage.

A stainless-steel safety push handle with rugged synthetic side corner guard elements, which also protects the operating elements, is located on the operator side. With a tube diameter of 28 mm, it enables ergonomic pushing of the dispenser.

Two insulated hooded covers made of EPP (expanded polypropylene) significantly decrease the heat loss and prevent the dishes from being soiled. The EPP hooded covers are also characterised by their light weight and extreme sturdiness and impact resistance.

Using the retaining clips included in the plate dispenser's scope of delivery, the EPP cover can be securely stored below the push handle when dispensing the plates. For this, the retaining clip is clicked into the front of the dispenser's push handle and the hooded cover can be hooked in below it. The retaining clip is designed so that it can securely hold both the EPP hooded cover included in the scope of delivery and a polycarbonate hooded cover which is optionally available.

The dispenser is mobile on corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, castor diameter 125 mm).

Rugged synthetic corner guards at all four corners of the dispenser protect the unit and furniture from damage.

### Accessories/options

- Transparent polycarbonate hooded cover

- Refilling signal

(The refilling signal shows personnel when the plates in the corresponding tube are running low.)

- All-round bumper rail

- 4 additional plate guides

(Using the additional plate guides, 4 stacks of small dishes such as soup dishes or salad and dessert bowls can be stacked in the plate tube. Also, the additional plate guides support stacks of unusually shaped plates)

- Fold-down shelf on short side

- GN hanging frame on the short side

(Offers space for 3 GN 1/6 Gastronorm containers)

- Cleaning drawer

- Stainless-steel body powder-coated in a variety of colours.

- For additional castor models, see full price list

Technical data

Material:		1.4301 / 1.4016

Weight:		Approx. 41 kg

Max. Load:		80 kg

Capacity:		approx. 80 plates (depending on the stacking height)

Dish dimensions:		round: 18-33 cm dia.

Rectangular: max. 28x28 cm

Number of tubes:		1

Hooded covers:			Expanded polypropylene (EPP)

Stacking height with
hooded cover:			800 mm

Stacking height w/o
hooded cover:			785 mm

Temperature range:		Adjustable from +30°C to

+110°C

Connected load:			220-240 V / 50-60 Hz / 1.5 kW

Protection type:			IPX5

Emissions: 			The workplace-specific noise level for the unit is less than 70 dB(A)

### Special features

- Large, square plate tube for a high level of flexibility in loading various plate sizes and plate shapes

- Particularly flat, stacking platform with jam-proof design

- High guide poles to hold the plates securely, even without hooded covers

- Removable plate tube

- Simple spring setting due to the Easy-Set System

- Insulated EPP hooded cover for less heat loss and high level of energy efficiency

Make

Manufacturer:	B.PRO

Type:	TS-UH1 18-33

Order no.:	575626