BASIC LINE UK-3 Kids – Cold buffet with active convection cooling 3 x GN 1/1 with low module height for children's catering

Dimensions:

Width:					1,255 mm

Depth:					  690 mm

Depth 1:				  775 mm

(= with folded-down tray slide on customer side)

Depth 2:				  990 mm

(= with folded-up tray slide on customer side)

Height of top surface:		  750 mm

Height incl. bridge attachment:	1,155 mm

The final module equipment depends on the selected configuration.

Basic module:

Mobile module, based on a sturdy, self-supporting sheet metal structure, installed ready to plug in, with approx. 2 m long connection cable and plug.

Top surface:

The convection-cooled cooling tray with CFC-free insulation is mounted into the 40 mm-high micro-polished CNS stainless-steel top surface (chrome-nickel-steel) with smooth bevelling on all sides. The cooling tray is designed to hold up to three 1/1-GN-150 containers or their subdivisions. The CNS inner section of the cooling tray consists of individual, easily removable segments; underneath this is the large-area finned evaporator with the fan. The finned evaporator can be folded up with a locking bar and locked in place during cleaning and service work. The lower tray bottom is embossed with a downhill slope toward the centre of the tray. The condensation water drains off into a manually emptied condensation-water catch tray.

The on/off switch for the refrigeration unit, the electronic cooling point control with digital display and the optional LED lighting are installed in the operator-side recessed panel on the refrigeration unit compartment to protect it against impact.

Underframe:

The side faces and the trim panel in front of the well section on the customer and operator sides are made of powder-coated thin sheet electrolytically galvanised on both sides. The installation compartment for the fully hermetically enclosed refrigeration unit is made of the same material. The refrigeration unit is conveniently accessible from the operator side for cleaning and servicing by unscrewing the installation compartment cover. Standard colour of the basic buffet below the stainless-steel top surface:

Traffic grey B (RAL 7043)

The following colours are alternatively available for the underframe: Please refer to Accessories/B.PRO colour options.*

The castors are mounted on the bottom surface of the side faces: 2 twin steering castors with brakes on the operator side, 2 twin steering castors on the customer side, castor diameter 75 mm.

Accessories/options:

- Body colour: Side faces & panels powder-coated in B.PRO colours.*

- Customer-side front panelling, between the side faces, easy to remove and fit

- produced from thin sheet, electrolytically galvanised on both sides, powder-coated in B.PRO BASIC LINE colours.*

- or produced from particle board, faced with Resopal laminated sheet material

- Operator-side front panelling, designed as hinged doors between the side faces.

- produced from thin sheet, electrolytically galvanised on both sides, powder-coated in the B.PRO BASIC LINE colours * or faced with Resopal laminated sheet material.

- B.PRO colours:*

Merlot red 19-1531 TPG

Sea blue 19-4234 TPG

Petrol green 18-5112 TPG

Candy red 17-1562 TPG

Neomint 15-5718 TPG

Signal white, RAL 9003

Stone grey, RAL 7030

Traffic grey B, RAL 7043

Graphite black, RAL 9011

Broom yellow, RAL 1032

Lime, Pantone 382 C

Apple green, Pantone 370 C

- Resopal laminated sheet material in over 180 "Colours" or "Woods" Resopal decors

- Standard bridge attachment with sneeze guard.

The smooth-surfaced, fully panelled bridge attachment, produced from micro-polished CNS stainless-steel sheet is fitted in the centre of the top surface.  CNS stainless-steel brackets for attaching the sneeze guard glass are fitted to the left and right of the bridge attachment. The brackets are U-shaped to protect the glass from damage on the short sides. Produced from toughened safety glass, the sneeze guard is fitted in such a way that it slopes towards the customer side and provides a pass-through opening. Pass-through height: 280 mm.

- Highline bridge attachment with toughened safety glass sneeze guard is attached so that it is horizontal to the top surface on the customer side and provides a pass-through opening. Pass-through height: 395 mm.

- Operator-side sneeze guard produced from toughened safety glass.

Pass-through height with standard attachment: 280 mm

Pass-through height with Highline attachment: 395 mm

- Customer-side sneeze guard produced from toughened safety glass, without pass-through, drawn down to the top surface.

- LED lighting with 3 LED spotlights is installed in the bridge attachment. Spotlights can be switched off and on separately.

- Tray slide produced from CNS stainless-steel round tube (tube 25 mm in diameter) on customer side and/or operator side, folds down. With child safety lock. Fixed locking using locking pins.

- Tray slide produced from CNS stainless-steel sheet on customer side and/or operator side, folds down. With child safety lock. Fixed locking using locking pins.

- Tray slide produced from Multiplex panel, faced with Resopal on customer side and/or operator side, folds down. With child safety lock. Fixed locking using locking pins.

- Plate slide produced from CNS stainless-steel sheet on customer side and/or operator side, folds down. Flush with the top surface at a height of 750 mm. With child safety lock. Fixed locking using locking pins.

- Plate slide produced from Multiplex panel, faced with Resopal on customer side and/or operator side, folds down. Flush with the top surface at a height of 750 mm. With child safety lock. Fixed locking using locking pins.

- Shelf produced from CNS stainless-steel round tube (tube 25 mm in diameter) on short side, right and/or left, folds down. With child safety lock. Fixed locking using locking pins.

- Shelf produced from CNS stainless-steel sheet on short side, right and/or left, folds down. With child safety lock. Fixed locking using locking pins.

- Shelf produced from Multiplex panel, faced with Resopal on short side, right and/or left, folds down. With child safety lock. Fixed locking using locking pins.

- Switch panel cover. To cover switches and operating elements.

- Additional socket outlets. Two additional socket outlets with 230 V Schuko socket each on operator side, installed in the refrigeration unit compartment in the right-hand side face. The maximum permitted power input for socket outlets to connect external units depends on the unit equipment.

- Mains connection with 16A CEE plug, 400 V.

- Upwards cable routing, block length: 450 mm, extended approx. 2 m

- CNS stainless steel base bottom

- Module connection set for permanently interconnecting two buffets, including profile panel to cover the gap between the two modules.

- Stainless-steel castors, 125 mm in diameter, 4 steering castors, 2 of which have brakes. Total height is increased by 60 mm, underframe height is then 810 mm

- Adjustable stainless-steel feet (instead of castors)

- Stainless-steel skirting panels on customer side/short side right/left (can only be ordered in conjunction with adjustable feet)

- Stainless-steel cabinet shelf

- Stainless steel GN hanging frame, for subdividing GN containers

Technical data:

Material:	Stainless steel (AISI 304), micro-polished, combined with

powder-coated thin sheet,

electrolytically galvanised on

both sides.

Weight:		Depends on equipment

Capacity:		Max. 3x GN 1/1-150

Temperature

range:	The temperature is factory pre-set to +7 °C and can be set to values between +2 °C and +15 °C* when the ambient temperature is max. +32 °C

Refrigerant:	Propane R290

Refrigerant mass:	110 g

Refrigerating

capacity:	0.66 kW at t0 = –10 °C and 
tu = +32 °C

Climate class:	4

Protection type: 	IPX 2

Connected load:	220–240 V/1N PE/50 Hz

380–415 V/3N PE/50 Hz

Emissions:	The workplace-specific

noise level of the unit is

less than 70 dB(A)

The module's connected load depends on the optional accessories.

*The temperature is reached at the geometric centre of the cooling tray.

Special features:

- The convection-cooling well is now made of individual segments that are easy to remove and clean in the dishwasher.

- Integrated ventilation grates ensure optimum cool air circulation and prevent food from falling into gaps during serving.

- GN containers can be positioned flexibly or displayed at an angle.

- Electronic cooling point control with digital display

- Whisper-quiet, energy-saving refrigeration unit with sustainable refrigerant R290

- Toughened safety glass sneeze guard

- Micro-polished surface made of stainless steel

- Two-section safety drain valve

- Fold-down tray and plate slides produced from CNS stainless-steel round tube, CNS stainless-steel sheet and Multiplex with Resopal lining.

- Large selection of colour scheme and customisation options

Make:

Manufacturer:		B.PRO

Model:         		BASIC LINE UK-3 Kids

Order no.		393 180