# 2 SHE 21-26 – Plate dispenser

Dimensions

Length:						  885 mm

Width:						  520 mm

Height:						  937 mm

Height (with hooded cover):		1027 mm

Operating height:				  900 mm

## Design

The dispenser housing is made of high-quality, corrosion-resistant stainless steel. The body is double-walled and features an insulated, closed design. The surface is micro-polished.

The synthetic guide rods can be used to adjust the two stacking tubes to hold round dishes with a diameter of 21-26 cm.

To ensure a consistently even serving height, the spring tension can be regulated depending on the stacked items by hooking or unhooking the tension springs.

The lower-lying operating panel has an on/off switch, indicator light, helix cord, dummy plug socket and temperature control on the short side. The temperature control ensures continuous temperature adjustment of the heating.

The plate dispenser is fitted with a powerful stainless-steel heating element.

A stainless steel safety push handle with side synthetic (polyamide) corner guard elements, which also protects the switch elements, is located on the operator side.

The dispenser can be moved on its corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm in diameter). Solid synthetic (polyamide) corner guards at all four corners protect against damage.

Two hooded covers made of transparent synthetic decrease the heat loss and prevent the dishes from being soiled.

### Accessories/options

- Lid lock on the top surface

- See full price list for additional castor models and accessories

Technical data

Material:	1.4301 / 1.4016

Weight:	45 kg

Capacity:	approx. 120 plates

Inside dimensions:	210 - 260 mm dia.

Number of tubes:	2

Hooded tube cover:	Polycarbonate

Stacking height with

hooded cover:		670 mm

Stacking height w/o

hooded cover:		620 mm

Unit interior:		Temperature adjustable
from +30°C to

+110°C

Connected load:		220-240 V/50 Hz/1.8 kW

Emissions:		The workplace-specific noise level for the unit is less than 70 dB(A)

### Special features

- Protected against splashed and sprayed water (IPX 5)

- Spring tension can be changed

- Synthetic guide rails protect china from wear

- As per DIN 18665, Part 6

Make

Manufacturer:	B.PRO

Model:	2 SHE 21-26

B.PRO INMOTION

Order No.:	574 836