# ARW 9x6-2 clearing trolley

Dimensions

Length:				1000 mm

Width:			  650 mm

Height:			1030 mm

Inside dimensions:

Height between shelves:		  585 mm

Shelves:		   900 x 550 mm

Model

The clearing trolley is made of stainless steel, 
AISI 304. There are two welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the clearing trolley’s stability. Closed stainless-steel railing on three sides is fitted above the top shelf (height: 
100 mm), which is welded to the round tube.

A noise-insulating antidrumming mat is positioned beneath the shelves.

The clearing trolley is mobile on air-identical corrosion-resistant synthetic castors as per DIN 18867, Part 8 (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Stainless-steel waste container AFB 4 x 2 x 2.5, with mounting hooks, 18.5 litres (order no. 555 509)

- Stainless-steel cutlery holder BGN 1/3-150, with mounting hooks, 110 pieces of cutlery

(order no. 555 510)

- Shelf reinforcing profiles, increases the max. load per shelf by 10 kg, the load-bearing capacity of the overall SW remains unchanged, for easy self-assembly (order no. 575 531)

Technical data

Material:			Stainless steel,

polyethylene (PE)

Weight:		18 kg

Load-bearing capacity’
per trolley:		120 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		2

### Special features

- All-round profile edge on the shelves to reinforce the stability of the clearing trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:		B.PRO

Model:                                ARW 9x6-2

Order no.		575 492