BASIC LINE WQ-3 Kids - Hot Buffet 3 x GN 1/1 Diagonal with low module height for children's catering

Dimensions:

Width:					1935 mm

Depth:					  690 mm

Depth 1:				  775 mm

(= with folded-down tray 
slide on customer side)

Depth 2:				  990 mm

(= with folded-up tray slide
 on customer side)

Height of top surface:		  750 mm

Height incl. bridge attachment:	1155 mm

The final module equipment depends on the selected configuration.

Basic module:

Mobile module, based on a sturdy, self-supporting sheet metal structure, installed ready to plug in, with approx. 2 m long connection cable and plug.

Top surface:

Three deep-drawn bain-marie wells with individual regulation and switching, as well as a stamped fill-level marking, are seamlessly welded into the 40 mm high, micro-polished CNS stainless steel (chrome-nickel steel) top surface with smooth bevelling on all sides. The wells are welded in across the width for ergonomic removal of the food. The bain-marie wells are designed to hold 1/1-GN-200 containers or their subdivisions. The wells can be indirectly heated wet or dry, and the power consumption per well is 0.7 kW. Each well is equipped with a ¾" drain, and all wells are connected to a common drain pipe with a central safety drain valve. The safety drain valve is designed in two sections to prevent accidental opening.

The on/off switches and thermostats for the individually switchable bain-marie wells are installed in the angled well trim panel (switch panel cover) on the operator side to protect them against jolts/impact.

Underframe:

The side faces and the trim panel in front of the well section on both the customer and operator sides are produced from powder-coated thin sheet that is electrolytically galvanised on both sides. Standard colour of the basic buffet below the CNS stainless-steel top surface:

Traffic grey B (RAL 7043)

The following colours are alternatively available for the underframe: Please refer to Accessories/B.PRO colour options.*

A CNS stainless-steel base bottom, for example to hold B.PROTHERM BPT 420 K / KBUH / KBRUH, is provided as standard.

The castors are mounted on the bottom surface of the side faces: 2 twin steering castors with brakes on the operator side, 2 twin steering castors on the customer side, castor diameter 75 mm.

Accessories/options:

- Body colour: Side faces & panels powder-coated in B.PRO colours.*

- Customer-side front panelling, between the side faces, easy to remove and fit

- produced from thin sheet, electrolytically galvanised on both sides, powder-coated in B.PRO BASIC LINE colours.*

- or produced from particle board, faced with Resopal laminated sheet material

- Operator-side front panelling, designed as hinged doors between the side faces.

- produced from thin sheet, electrolytically galvanised on both sides, powder-coated in the B.PRO BASIC LINE colours * or faced with Resopal laminated sheet material.

- B.PRO colours:*

Merlot red 19-1531 TPG

Sea blue 19-4234 TPG

Petrol green 18-5112 TPG

Candy red 17-1562 TPG

Neomint 15-5718 TPG

Signal white, RAL 9003

Stone grey, RAL 7030

Traffic grey B, RAL 7043

Graphite black, RAL 9011

Broom yellow, RAL 1032

Lime, Pantone 382 C

Apple green, Pantone 370 C

- Resopal laminated sheet material in over 180 "Colours" or "Woods" Resopal decors

- Standard bridge attachment with sneeze guard.

The smooth-surfaced, fully panelled bridge attachment, produced from micro-polished CNS stainless-steel sheet is fitted in the centre of the top surface.  CNS stainless-steel brackets for attaching the sneeze guard glass are fitted to the left and right of the bridge attachment. The brackets are U-shaped to protect the glass from damage on the short sides. Produced from toughened safety glass, the sneeze guard is fitted in such a way that it slopes towards the customer side and provides a pass-through opening. Pass-through height: 280 mm.

- Operator-side sneeze guard produced from toughened safety glass. Optionally available with rounded glass.

Pass-through height with standard attachment: 280 mm

Pass-through height with Highline attachment: 	395 mm

- Customer-side sneeze guard produced from toughened safety glass, without pass-through, drawn down to the top surface.

- Fold-down sneeze guard for standard bridge attachment. Simply fold the sneeze guard up and down without the need for tools.

- Three individually switchable ceramic radiators with a power consumption of 200 W each. Fitted in the bridge attachment. The ceramic radiators employ a close-meshed CNS stainless-steel grid plate to prevent direct contact.

- LED lighting with 3 LED spotlights (or 4 LED spotlights with radiant heater/LED combination) are fitted in the bridge attachment. Spotlights can be switched off and on separately from the radiant heaters.

- Tray slide produced from CNS stainless-steel round tube (tube 25 mm in diameter) on customer side and/or operator side, fold-down. With child safety lock. Fixed locking using locking pins.

- Tray slide produced from CNS stainless-steel sheet on customer side and/or operator side, fold-down. With child safety lock. Fixed locking using locking pins.

- Tray slide produced from Multiplex panel, faced with Resopal on customer side and/or operator side, fold-down. With child safety lock. Fixed locking using locking pins.

- Plate slide produced from CNS stainless-steel sheet on customer side and/or operator side, fold-down. Flush with the top surface at a height of 750 mm. With child safety lock. Fixed locking using locking pins.

- Plate slide produced from Multiplex panel, faced with Resopal on customer side and/or operator side, fold-down. Flush with the top surface at a height of 750 mm. With child safety lock. Fixed locking using locking pins.

- Shelf produced from CNS stainless-steel round tube (tube 25 mm in diameter) on short side, right and/or left, fold-down. With child safety lock. Fixed locking using locking pins.

- Shelf produced from CNS stainless-steel sheet on short side, right and/or left, fold-down. With child safety lock. Fixed locking using locking pins.

- Shelf produced from Multiplex panel, faced with Resopal on short side, right and/or left, fold-down. With child safety lock. Fixed locking using locking pins.

- Storage shelf produced from waterproof particle board, ABS edge banding 2 mm with multiplex printing. Corners rounded with radius. Available in 12 different Resopal colours:

0300-60 Infinity

9450-60 Teal

D421-60 Midori

0335-60 Orange

D362-60 Nile Green

0104-60 Traffic White

0150-60 Tin

0901-60 Black

D483-60 Yellow

D90-60 North Sea

0617-60 Bergamot

9429-60 Parrot

- Switch panel cover. To cover switches and operating elements.

- Additional socket outlets. Two additional socket outlets with 230 V Schuko each, installed on operator side inside the right side face. The maximum permitted power input for socket outlets to connect external units depends on the unit equipment.

- Mains connection with 16A CEE plug, 400 V.

- Upwards cable routing, block length: 450mm, extended approx. 2m

- Module connection set for permanently interconnecting two buffets, including profile panel to cover the gap between the two modules.

- Stainless-steel castors, 125 mm in diameter, 4 steering castors, 2 of which have brakes. Total height is increased by 60 mm, underframe height is then 810 mm

- Adjustable stainless-steel feet (instead of castors)

- Stainless-steel skirting panels on customer side/short side right/left (can only be ordered in conjunction with adjustable feet)

Technical data:

Material:	Chrome-nickel-steel 18/10 
(AISI 304), micro-polished, combined with

powder-coated, electrolytically galvanised thin sheet on both sides.

Weight:		Depends on equipment

Capacity:		Max. 3 x GN 1/1-200

Temperature range:	+30°C to +95°C

Connected load:	230 V / 16 A / 1N PE /

50-60 Hz

The module's connected load depends on the optional accessories.

Special features:

- Individually heated and controlled bain-marie wells for wet or dry operation

- Transversely arranged wells for easy and ergonomic serving of food for children and teenagers

- Separately switchable ceramic radiators over each bain-marie well

- Toughened safety glass sneeze guard

- Micro-polished CNS stainless-steel surface

- Two-section safety drain valve

- Fold-down tray and plate slides produced from CNS stainless-steel round tube, CNS stainless-steel sheet and Multiplex with Resopal lining and child safety lock.

- Switch panel cover to prevent guests form intentionally playing with the switches.

- Large selection of colour scheme and customisation options

Make:

Manufacturer:		B.PRO

Model:         		BASIC LINE WQ-3 Kids

Order no.:		392 821