Built-in bain-marie BME 1/1

Dimensions

Exterior dimensions:

Length:		371 mm

Width:		576 mm

Height:		250 mm

Design:

The BME 1/1 is fitted into Resopal top surfaces or similar and is supplied with corresponding securing elements. The well is made completely of stainless steel. The seamlessly deep-drawn well can be heated when wet or dry and features an all-round slosh guard.  Embossments in the base section ensure optimal stability. The base features a ½" drain fitting. The heating well is fitted beneath. The well is electrically wired with a connection for an external switch element with an indicator light and thermostat.

Accessories/options

Ball valve, Art. 098215

Additional insulation housing

Technical data:

Material:		Stainless steel

Capacity:		1 heat-retaining wells

Heat-retaining 
wells:			for GN 1/1 or its subdivisions

Temperature adjustable between +30 °C and +110 °C

Connected load:	230 V/50 Hz/0.7 kW

Drain:			½"

Make:

Manufacturer:	B.PRO

Model:	BME 1/1

Order-No.:	566260