# fShelf Trolley RWR 161-20

Dimensions

Length:				  663 mm

Width:			  734 mm

Height:			1801 mm

Inside dimensions:

Space between support rails:	    73 mm

Design

The shelf trolley consists of stainless steel, AISI 304. U-profile guide rails are welded between the sturdy 25 x 25 mm square tube. These are supplied with a tip safety as well as push-through protection on both sides for GN 2/1, 1/1 and 2/4.

A 20 x 10 mm square tube welded in at mid-height offset on both the front and back of the shelf trolley provides additional stability.

The shelf trolley is mobile on corrosion-resistant synthetic castors pursuant to DIN 18867, Part 8

(4 steering castors, 2 of which have brakes, castor diameter 125 mm). Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories/options

- CNS panelling, 2- or 3-sided

- Transport locking bar on both sides

- Push handle short side

Technical data

Material:			Stainless steel

Polyethylene (PE)

Material thickness

of the square tube:			1.25 mm

Material thickness

of the support rails:			1.25 mm

Weight:		25.0 kg

Load-bearing capacity

per trolley:		200 kg

Max. load-bearing

capacity per support pair: 	30 kg

Number of

support pairs:		20 pairs GN 1/1 or their subdivision

Capacity:			9 x GN 1/1-150 without lid

### Special features

- U-profile guide rails with push-through protection

Make

Manufacturer:		B.PRO

Model:                               RWR 161-20

Order No.:		568 498