# RWRA 852 Shelf Trolley

Dimensions

Length:				820 mm

Width:			613 mm

Height:			845 mm

Interior dimensions:

Space between support rails:	 73 mm

Design

The two-part shelf trolley consists of CNS 18/10 (AISI 304), Material No. 1.4301. U-profile guide rails are welded between the stable 25 x 25 mm square tubes. These are supplied with a tip safety as well as push-through protection on both sides for GN 2/1, 1/3 and 2/8. The shelf trolley is equipped with a smooth work surface (748 x 542 mm) which is turned down on 4-sides.

The shelf trolley can be moved on galvanized steel castors (4 steering castors, 2 of which have locking brakes, castor diameter 125 mm). Solid synthetic (polyamide) wall guards at all four corners protect against damage.

Accessories and options

- CNS paneling, 2 or 3-sided

- Transport circlip, double-sided

Technical data

Material:			Chrome-nickel-steel 18/10 (AISI 304), Polyamide (PA)

Material thickness of the

square tube:				1.5 mm

Material thickness of the

support rails:				1.2 mm

Weight:			22 kg

Load-bearing capacity

per trolley:			180 kg

Max. load-bearing

capacity per support pair: 		30 kg

Number of support pairs:		16 (2 x 8) pairs GN 1/1 or their subdivision

Capacity:			8 x GN 1/1-150 without lid

### Special features

- U-profile guide rails with tip safety and push-through protection on both sides

- 2-part design with work surface

- For rolling under open worktables

Make

Manufacturer:		B.PRO

Model:                               RWRA 852

Order No.:		572 929