# Shelf Trolley RWR SK-161

Dimensions

Length:				  662 mm

Width:			  733 mm

Height:			1645 mm

Interior dimensions:

Space between support rails (top):  115 mm

Interior dimensions:

Space between support rails (bottom):   73 mm

Design

The shelf trolley consists of CNS 18/10 (AISI 304), Material No. 1.4301. 5 pairs of U-profile guide rails are welded between the stable 25 x 25 mm square tubes, mounted diagonally at approx. 10°. These are supplied with a tip safety as well as push-through protection on both sides. Below these are 9 pairs of horizontally mounted support rails, which are supplied with a tip safety and push-through protection on both sides.

5 serving doors for salads and desserts are attached at the front of the top 5 pairs of support rails.

The shelf trolley can be moved on galvanized steel castors (4 steering castors, 2 of which have locking brakes, castor diameter 125 mm). Solid synthetic (polyamide) wall guards at all four corners protect against damage.

Technical data

Material:			Chrome-nickel-steel 		18/10 (AISI 304),

Polyamide (PA)

Material thickness of

the square tube:			1.5 mm

Material thickness of the

support rails:			1.2 mm

Weight:		30 kg

Load-bearing capacity

per trolley:		200 kg

Max. load-bearing

capacity per support pair: 	30 kg

Number of support

pairs (diagonal):		5

Number of support

pairs (horizontal):		9

Capacity:		14 x GN 2/1-20, or

14 x BZG 2/1-20

### Special features

- 9 pairs of horizontally mounted U-profile guide rails with tip safety and push-through protection on both sides

- 5 pairs of diagonally (approx. 10°) mounted support rails with tip safety and push-through protection on one side

- Salad and dessert serving trolley

Make

Manufacturer:		B.PRO

Model:                               RWR SK-161

Order No.:		572 943