# SW 10x6-3 BASIC serving trolley

Dimensions

Length:				1100 mm

Width:			  700 mm

Height:			  950 mm

Inside dimensions:

Height between shelves:		  275 mm

Shelves:	   	 1000 x 600 mm

Model

SW 10x6 serving trolley in basic design.

The serving trolley is made of stainless steel, 
AISI 304. There are three welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the serving trolley’s stability.

A noise-insulating antidrumming mat is positioned beneath the shelves.

The serving trolley is mobile on galvanised steel castors (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Stainless-steel cutlery attachment CAT 5, including 5 GN 1/4-150 cutlery holders with transparent, half-flap Plexiglas top surface, 
500 pieces of cutlery. For easy mounting on tube clamps (order no. 574 772)

- Stainless-steel waste container AFB 4 x 2 x 2.5, with mounting hooks, 18.5 litres (order no. 555 509)

- Stainless-steel cutlery holder BGN 1/3-150, with mounting hooks, 110 pieces of cutlery

(order no. 555 510)

- All-round railing, for easy self-assembly (order no. 575 525)

- Railing on 3 sides, for easy self-assembly (order no. 575 527)

- Shelf reinforcing profiles, increases the max. load per shelf by 10 kg, the load-bearing capacity of the overall SW remains unchanged, for easy self-assembly (order no. 575 532)

Panelling sets:

- Stainless-steel panelling on 3 sides, for hanging on the serving trolley, order no: 375451

- Panelling on 3 sides with front stainless-steel hinged doors for easy mounting on serving trolley, order no.: 375452

- Thin-sheet panelling on 3 sides, electrolytically galvanised on both sides, powder-coated in various colours; for hanging on the serving trolley; order no.: 375455

- Thin-sheet panelling on 3 sides with front hinged doors, electrolytically galvanised on both sides, powder-coated in various colours; for easy mounting on serving trolley, order no.: 375456

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		21.2 kg

Load-bearing capacity
per trolley:		120 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		3

### Special features

- All-round profile edge on the shelves to reinforce the stability of the serving trolley and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:		B.PRO

Model:                                SW 10x6-3 BASIC

Order no.		573 489