# MCS 10x6-2 Kids mobile clearing station

Dimensions

Length:				1100 mm

Width:			  700 mm

Height:			  800 mm

Inside dimensions:

Height between shelves:		 435 mm

Shelves:		1000 x 600 mm

Model

The MCS 10x6-2 Kids mobile clearing station based on a 10x6 serving trolley in a basic design.

The mobile clearing station is made of stainless steel, AISI 304. There are two welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the clearing station’s stability.

A noise-insulating antidrumming mat is positioned beneath the shelves. The top shelf contains a cut-out in which two Gastronorm containers with 
GN-B 1/3-150 U-handles are positioned. The clearing station is equipped with 4 gusset plates on the upper shelf for additional stability.

The clearing station is mobile on corrosion-resistant synthetic castors as per DIN 18867, Part 8 
(4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Drip tray with handles, for glass racks, 
500 x 500 mm, made of stainless steel, placed on the top shelf (order no.: 574 779)

- Stainless-steel storage shelf, fitted at children's height (700 mm) without tools

(Order no. 574 780)

- Hanging frame for GN containers, made of stainless steel

(Order no. 574 781)

Offers space for up to 3 Gastronorm containers GN 1/6

Panelling and privacy shield:

- Thin-sheet panelling on 3 sides, electrolytically galvanised on both sides, powder-coated in various colours; for hanging on the clearing station, order no.: 392574

- Thin-sheet privacy shield, electrolytically galvanised on both sides, powder-coated in various colours; for easy mounting on tube clamps, order no.: 382662

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		Approx. 28 kg

Load-bearing capacity
per trolley:		140 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		2

Max. capacity:		40 people

### Special features

- All-round profile edge on the shelves to reinforce the stability of the clearing station and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer:			B.PRO

Model:	MCS 10x6-2 Kids

Order no. 			575623