# UNI 59/ 29 - Universal dispenser Kids

Dimensions

Length:					775 mm

Width:					520 mm

Height:					794 mm

Height (with optional hooded cover)	878 mm

Working height:					750 mm

Model

Design

The universal dispenser is made entirely of stainless steel The surface is micro-polished.

The body is single-walled and compact in a closed design. The universal dispenser has continuous powder-coated inner panelling to prevent streaks on the dishes.

Thanks to the platform's grid division, a wide variety of dish types can be stacked using the 6 variably divisible plastic guide rails.

The stacking platform runs smoothly and is secured against tipping and tilting by guides with ball-bearing castors.

To ensure a constant, even serving height, the tension springs can be to adjust the spring tension to hold the particular stacked items in question.

A stainless-steel safety push handle with synthetic (polyamide) side corner guard elements is attached to the operator side.

The dispenser can be moved on its corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm in diameter). Solid synthetic (polyamide) corner guards at all four corners protect against damage.

### Accessories/options

- Hooded cover (642 x 342 mm), polycarbonate

Stacking height with hooded cover:	   530 mm

Stacking height without hooded cover: 390 mm

- See full price list for additional accessories and castor models

Technical data

Material:		Stainless steel

Weight:		Approx. 45 kg

Inside dimensions:	590 x 290 mm

Capacity:		Dependent on type of dishes

### Special features

- Spring tension can be changed

- variable settings thanks to 6 guide rods

- As per DIN 18665, Part 6

Make

Manufacturer:			B.PRO

Model:				UNI 59/ 29 Kids

Order no.			575 629