Gastronorm – Container GN 1/1-40

Dimensions

Exterior dimensions:

Length:		530 mm

Width:		325 mm

Depth:		  40 mm

Inside dimensions:

Length:	505 mm

Width:		300 mm

Base radius:		R 15 mm

Corner radius:	R 80 mm

Design:

This Gastronorm container is made of AISI 304 stainless steel and is seamlessly deep-drawn. The all-round, highly pronounced stacking collar ensures that containers are stacked or unstacked evenly and also helps prevent jamming. Exceptionally large corner and base radii ensure that food is completely used up while also making cleaning easy. The all-round edge embossment makes the entire edge more stable and more rigid. The GN container features drop-shaped embossments on its edges. The functional corners ensure sturdy corner sections and extreme shock resistance and allow food to be tipped out in the required quantity.

Technical data:

Material:	AISI 304 stainless steel

Standards:	DIN EN 631

Capacity:	up to 20 mm below edge: 	2.7 litres

up to 3 mm below edge:

5.1 litres

Weight:	1,25 kg

# Special features:

- Large corner radii (side wall to side wall)

- Increased base radii (base to side wall)

- Container edge with functional corner

Make:

Manufacturer:	B.PRO

Model:	GN 1/1-40

Order No.:	1 550 064