Gastronorm – GDD 2/3 sealing lid

Dimensions

Length:	354 mm

Width:	325 mm

Height:	  20 mm

Model

The Gastronorm sealing lid consists of CNS 18/10 (AISI 304), material number 1.4301 and is seamlessly deep-drawn. The all-round profile edge ensures an optimum fit of the lid on the respective Gastronorm container. The lid has been provided with raised areas on the surface up to the edge area for increased rigidity and stability. The gripping depression with tack-welded handle for easy lifting/setting down of the lid is located in the center of the GN lid.

The GN lid has been provided with a food-safe silicone seal for slosh-proof transport. The silicone seal is resistant to temperatures up to +180 °C and form-fitting with the GN lid edge. It consists of a permanent, physical bond between the stainless-steel lid and the silicone seal. The lid also features a ventilation hole (5 mm diameter). The ventilation hole ensures pressure compensation in the container and allows easy lifting/setting down of the lid.

Technical data

Material:	CNS 18/10 (AISI 304) - 1.4301

Silicone, color RAL 5000

Standards:		  DIN EN 631

Weight:		  0,8 kg

### Special features

- Form-fitting seal

- Permanent, physical bond between stainless-steel lid and silicone seal

Make

Manufacturer:        	B.PRO

Type:               	GDD 2/3

Order No.:         	  1550662