# Gastronorm – Container GN-B 1/3-100

Dimensions

Exterior dimensions:

Length:			325 mm

Width:			176 mm

Depth:			100 mm

Inside dimensions:

Length:		300 mm

Width:			151 mm

Total height, including U-handle:		108 mm

Base radius:			R 15 mm

Corner radius:			R 50 mm

Design

This Gastronorm container is made of AISI 304 stainless steel and is seamlessly deep-drawn. The all-round, highly pronounced stacking collar ensures that containers are stacked or unstacked evenly and also helps prevent jamming. Exceptionally large corner and base radii ensure that food is completely used up while also making cleaning easy. The all-round edge embossment makes the entire edge more stable and more rigid. The GN container features drop-shaped embossments on its edges. The functional corners ensure sturdy corner sections and extreme shock resistance and allow food to be tipped out in the required quantity.

Universal U-handles

This Gastronorm container features universal U-handles. The ergonomically shaped U-handles enables users to easily lift and carry the container with a single hand, meaning two containers can be carried at the same time. Thanks to the offset two-point support, the handles are not attached directly to the container edge, so they cannot become hot. This means the container can be easily placed in or removed from bain-marie basins.

Technical data

Material:	AISI 304 stainless steel

Standards:	DIN EN 631

Capacity:	up to 20 mm below edge: 	3.0 litres

up to 3 mm below edge:

3.8 litres

# Special features

- Large corner radii (side wall to side wall)

- Increased base radii (base to side wall)

- Container edge with functional corner

- Universal D-handles for optimum handling

Make:

Manufacturer:	B.PRO

Model:	GN-B 1/3-100

Order No.:	1 555 937