Gastronorm cooking insert, perforated, with folding handles – G-KEN G 1/1-145

Dimensions

Exterior dimensions:

Length:		530 mm

Width:		325 mm

Depth:		145 mm

Inside dimensions:

Length:		500 mm

Width:		295 mm

Base radius:		R 25 mm

Corner radius:		R 77.5 mm

# Design

This Gastronorm cooking insert is made of AISI 304 stainless steel and is seamlessly deep-drawn. The base and side walls feature holes 4 mm in diameter while the holes in the corner radii are 3 mm in diameter (space between holes = 6 mm). The hole pattern designed especially for the cooking insert ensures shorter cooking times and faster cooling or draining for food. Exceptionally large corner and base radii ensure that food is completely used up while also making cleaning easy.

# Folding handle

The Gastronorm cooking insert features two folding handles, on the right and left sides. The folding handles make it easier to take the G-KEN out of the Gastronorm container. The handles can be folded inside, so that several filled containers can be stacked one on top of the other or handles do not get in the way while cooking.

Technical data

Material:	AISI 304 stainless steel

Standards:	DIN EN 631

Capacity:	up to 20 mm below edge: 	16.3 litres

up to 3 mm below edge:

19.0 litres

# Special features

- Large corner radii (side wall to side wall)

- Increased base radii (base to side wall)

- Base, corners and sides perforated

- Folding handles for safe lifting and carrying

Make

Manufacturer:	B.PRO

Model:	G-KEN G 1/1-145

Order No.:	550 972