# B.PROTHERM 320 ECO

Dimensions

Length:				630 mm

Width:			415 mm

Height:			345 mm

Description

The food transport container is made of high-quality, physiologically safe, and food-resistant polypropylene.

The double-walled synthetic container and lid are insulated with PUR foam.

In the inside container, there are indentations around the edge for inserting support bars securely. The practical clamping lid is easy to open and to shut. The safety device incorporated into the reinforced lid prevents the GN container from opening accidentally during transport.  A labelling card holder is incorporated into the short side of the container for easy identification of the food inside. The container is completely dishwasher safe to max. +96°C.

The interior is temperature resistant from -25 to +100°C.

For use with GN containers, max. size GN 1/1 – 200 mm deep or their subdivisions, inserted from above. Also suited for use with Gastronorm containers with top handles thanks to an all-round recess in the lid. All B.PROTHERM containers can be stacked on top of each other – even when open without lids.

The average temperature loss in closed containers is approx. 1.5°C per hour for warm food; cold foods warm up approx. 0.5°C per hour (complies with DIN EN 12571).

### Accessories / options

- various transport trolleys and dollies—please see the general price list

- eutectic plate -3°C (order no. 569 315)

- GN support bars for variable use of GN containers (ST 3 order no. 550 650, ST 5 order no. 550 651)

- menu cards and accessories--please see the general price list

Technical data

Material:	polypropylene (PP),

polyamide (PA)

Insulation material:	PUR foam, CFC-free

Capacity:	GN 1/1-200 or their

subdivisions

Weight:	6,58 kg

### Special features

- dishwasher safe to max. +96°C

- recyclable

- moulded handles and skids

Make

Manufacturer:			B.PRO

Model:	BPT 320 ECO

B.PROTHERM

Order no.			564 650