# CEBH 50/ 50 – basket dispenser

Dimensions

Length:					700 mm

Width:					691 mm

Height:					785 mm

##### Description

Structure

The build-in basket dispenser is made entirely of CNS 18/10 (AISI 304) with a specially ground matte surface finish (“microlised” surface).

The dispenser has a single-walled body in a closed and compact design for installation from below. The base of the dispenser has an opening for cleaning the interior. The body is insulated to prevent heat loss.

The baskets are placed on a platform with all-round turned up edges that hold them in place.

The platform is supported by tension springs. By hanging or unhanging individual springs, the serving height can be adjusted according to the weight of the objects stacked.

The temperature control allows for continuous setting of the forced-air convection heating.

### Accessories / options

- steel wire baskets, plastic-coated

500 x 500 x 75 mm or 500 x 500 x 115 mm

- stainless steel baskets

500 x 500 x 75 mm or 500 x 500 x 115 mm

- coloured panelling

- installation from above

- for other castor variations, please refer to our general price list

Technical data

Material:		CNS 18/10 (AISI 304)

Weight:		62.5 kg

Interior dims.:		520 x 520 mm

Capacity:		6 baskets, each 115 mm high,

9 baskets, each 75 mm high

Stacking height:		540 mm

Interior:		temperature adjustable from
		+30°C to +110°C

Voltage / wattage:		220-240V / 50Hz / 1.8kW

Emissions:		The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- protected against splash and jet water (IPX 5)

- adjustable spring tension

- homogenous heat distribution due to forced-air convection heating

- complies with DIN 18665, Part 6

Make

Manufacturer:			B.PRO

Model:				CEBH 50/ 50

B.PRO INMOTION

Order No.:			572 206