# B.PROTHERM 420 KBUH

Dimensions

Length:				692 mm

Width:			445 mm

Height:			470 mm

Model

The food transport container is manufactured with the thermoforming process and consists of high-quality, non-hazardous, food-resistant polypropylene.

The double-walled synthetic container and the door are filled with PUR foam for thermal insulation. Inside and outside containers are welded to be leak-free. The synthetic surface is smooth and non-porous.

Stainless-steel carrying handles with ergonomically shaped synthetic carrying handles are located on the long sides of the body recessed in depressions. The stainless-steel carrying handles enable wear-resistant use even with heavy loading.

The container is equipped with a hinged door that can be swivelled by 270° and removed if necessary. The patented door hinge ensures that the door can only be removed in a predefined position. This prevents accidental removal of the door and the related danger of injury.

The door is locked with a stainless-steel bent-clamp closure on the long side of the container, which engages in the door.

A device for holding marking cards is integrated on the door for marking the food in the container.

Two stacking skids are screwed onto the underside of the container and the top is provided with for screwed-on stacking corners. All B.PROTHERM containers can be stacked with one another.

The container is dishwasher-safe without the door up to a maximum temperature of +90°C.

All wearing parts (skids, stacking corners, handles, door hinges and door lock) can easily be replaced if service is required. All countersunk screws are covered with synthetic plugs for hygienic reasons.

The BPT 420 KBUH is loaded from the front. The inner body of the container is equipped with 11 pairs of moulded support ledges with a spacing of 31 mm for accepting Gastronorm containers.

The convection heating module is permanently integrated inside the hinged door. The electrical connection with a power plug coupling, which can be locked splash-water-proof with a special rotary lock, is located in the upper area of the door. The power on/off indicator is located next to this. This lights up as soon as the mains voltage is connected to the unit.

The temperature inside the container is limited to +90°C. The heater is equipped with an integrated overheating protection to prevent the food from being cooked further.

The power cable included loose with the container.

### Accessories/options

- Various transport trolleys and castors (see Complete Price List)

- Eutectic plate -3°C (order no. 568136)

- Eutectic plate -12°C (order no. 573332)

- Slide-in frame for subdivision of GN containers with a maximum depth of 150 mm

(Art. No. 564352)

- GN crossbars for variable insertion of GN containers of the size GN 1/4, GN 1/6, GN 1/9 in the slide-in frame (ST 3 Art. No. 550650,

ST 5 Art. No. 550651)

- Menu cards incl. accessories (see Complete Price List)

Technical data

Material:	Polypropylene (PP),

polyamide (PA),

CNS 18/10

Thermal

stability:	-25°C to +100 °C

Insulating material:	PUR foam, CFC-free

Ledges:	11 pairs of support ledges, spacing of 31 mm

Capacity:	2 x GN 1/1-150

Heat-up time:	in the empty container, a

temperature of approx. 75°C is reached in 45 min.

Weight:	14,61 kg

Connected load:	220-240 V AC/50-60 Hz/

200 W

Emissions:	The workplace-specific noise

Level of the unit is less than 70

dB (A).

Special features

- IPX 4 protection against splashed water

- VDE-GS tested

- Dishwasher-safe up to max. +90°C without door

- recyclable

- Power on/off indicator

- Heater with integrated overheating protection

- Patented door hinge (can only be removed in predefined position)

- Gripping depression at upper left in door for easy movement and optimal handling of the container

Make

Manufacturer:			B.PRO

Model:	B.PROTHERM

BPT 420 KBUH

Order No.:			573 515