# Plate dispenser TS-H2 18-33

Dimensions

Length:			                                1076 mm

Width:				520 mm

Height: (incl. plate guides)                        1030 mm

Height: (incl. hooded cover)                      1080 mm

Operating height:	900 mm

Model

The dispenser body is made of high-quality, corrosion-resistant stainless steel. The surface is micro-polished. The body is double-walled and insulated.

The plate dispenser is equipped with two square plate tubes. The plate tubes have a particularly flat stacking platform with jam-proof guides.

Using four Rilsan-coated plate guide rods, they can be adjusted to various plate sizes and plate shapes flexibly and without tools using a hole pattern on the bottom. For example, round plates with a diameter of 18 – 33 cm and rectangular plates up to a maximum of 28 x 28 cm can be stacked in each plate tube.

The plate guide rods project above the top surface of the dispenser. This means that more plates can be securely transported and unstacked with it and the dishes are still guided securely when the hooded covers have been removed.

The plate tubes can be completely removed from the dispenser to clean the dispenser easily and set the spring tension simply and ergonomically.

To ensure a consistently even serving height, the spring tension can be regulated depending on the stacked items by hooking or unhooking the tension springs.

This setting can be made quickly and effortlessly because of special cut-outs in the spring retainers.

The plate dispenser is equipped with a heating element made of AISI 304. The temperature control ensures continuous temperature adjustment of the heating. The plate dispenser also has an ECO control range. Within this control range, mid-range plate temperatures can be maintained in the most energy-efficient way.

The operating components and the helix cord are recessed in the switch panel on the short side to prevent damage.

An AISI 304 safety push handle with rugged synthetic side corner guard elements, which also protects the operating components, is located on the operator side. With a tube diameter of 28 mm, it enables ergonomic pushing of the dispenser.

Two hooded covers made of transparent synthetic (polycarbonate) decrease heat loss and prevent the dishes from being dirtied.

Using the retaining clips included in the plate dispenser's scope of delivery, the hooded covers can be securely stored when dispensing the plates. For this, the retaining clip is clicked into the front of the dispenser's push handle and the hooded covers can be hooked in below the handle.

The retaining clip is designed so that it can securely hold both the polycarbonate hooded covers included in the scope of delivery and an EPP hooded cover which is optionally available.

The dispenser is mobile on corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, castor diameter 125 mm).

Rugged synthetic corner guards at all four corners of the dispenser protect the unit and furniture from damage.

### Accessories/options

- Insulated hooded cover made of EPP, silver-grey

- refilling signal

(The refilling signal shows the personnel in time when the plates in the corresponding tube are running low.)

- All-round bumper rail

- 4 additional plate guides per tube

(Using the additional plate guides, 4 stacks of small dishes such as soup dishes or salad and dessert bowls can be stacked in the plate tube. Also, the additional plate guides support stacks of unusually shaped plates)

- Fold-down shelf on short side

- GN hanging frame on the short side

(Offers space for 3 Gastronorm containers GN 1/6)

- Cleaning drawer

- Body made of stainless-steel, powder-coated in various colours

- For additional castor models, see full price list

Technical data

Material:		1.4301 / 1.4016

Weight:		approx. 72 kg

Max. load:		160 kg

Capacity:		approx. 160 plates (depending on the stacking height)

Dish dimensions:		round: 18-33 cm dia.

Rectangular: 
		max. 28x28 cm

Number of tubes:		2

Hooded covers:			polycarbonate

Stacking height with

hooded cover:			800 mm

Stacking height w/o

hooded cover:			785 mm

Temperature range:		Temperature adjustable
			from +30°C to +110°C

Connected load:			220-240 V / 50-60 Hz / 1.2 kW

Protection type:			IPX5

Emissions:			The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- Large, square plate tubes for a high level of flexibility in loading various plate sizes and plate shapes

- Particularly flat, stacking platform with jam-proof design

- High guide rods to hold the plates securely, even without hooded covers

- Removable plate tubes

- Simple spring setting due to the Easy-Set System

- ECP control range to heat up the dishes and maintain its temperature of >/= 60°C energy-efficiently.

- Insulated hooded cover made of EPP (as an alternative to the polycarbonate hooded cover included in the scope of delivery) for less heat loss and a high level of energy efficiency

Make

Manufacturer:	B.PRO

Type:	TS-H2 18-33

Order No.:	573778