# CHV 66/ 54 – basket dispenser

Dimensions

Length:					1025 mm

Width:					  700 mm

Height:		  			  911 mm

Height (w/ optional domed lid):		1067 mm

Operating height:				  900 mm

Description

Structure

The basket dispenser is made entirely of CNS 18/10 (AISI 304) with a specially ground matte surface finish (“microlised” surface).

The dispenser has a double-walled body in a closed and compact design that protects the baskets from dirt.

The baskets are placed on a platform. The dispenser’s special construction prevents them from slipping inside the body.

The platform is supported by tension springs. By hanging or unhanging individual springs, the serving height can be adjusted according to the weight of the objects stacked.

A recessed control panel at the short side of the dispenser features an on-off switch, a control indicator, a helix cord, a dummy plug socket, and a temperature control. The temperature control allows for continuous setting of the forced-air convection heating. The basket dispenser also has an ECO control range. Within this control range, mid-range plate temperatures can be maintained in the most energy-efficient way.

The transparent domed lid protects the dispenser against heat loss and dirt.

A CNS safety push-pull handle with side bumper elements of plastic (polyamide) is mounted at the operator side. It also serves as a protection for the control elements.

The trolley is made mobile by corrosion-resistant synthetic castors (4 steering castors, 2 of which have brakes, 125 mm dia.).

Very robust synthetic corner guards (polyamide) at all four corners protect the dispenser from being damaged.

### Accessories / options

- steel wire baskets, plastic-coated

650 x 530 x 75 mm or 650 x 530 x 115 mm

- stainless steel baskets

650 x 530 x 75 mm or 650 x 530 x 115 mm

- domed lid (710 x 590 mm), polycarbonate

stacking height w/ domed lid:		680 mm

stacking height w/o domed lid:	540 mm

- coloured panelling

- for other castor variations, please refer to our general price list

Technical data

Material:		CNS 18/10 (AISI 304)

Weight:		75 kg

Platform:		660 x 540 mm

Capacity:		6 baskets, each 115 mm high,

9 baskets, each 75 mm high

Stacking height w/

domed lid:		680 mm

Stacking height w/o

domed lid:		540 mm

Interior:		temperature adjustable from
		+30°C to +110°C

Voltage / wattage:		220-240V / 50Hz / 1.8kW

Emissions:		The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- protected against splash and jet water (IPX 5)

- ECP control range to heat up the dishes and maintain its temperature of >/= 60°C energy-efficiently

- adjustable spring tension

- homogenous heat distribution due to

forced-air convection heating

- suitable for energy-consumption optimisation systems

- complies with DIN 18665, Part 6

Make

Manufacturer:			B.PRO

Model:				CHV 66/ 54

Order No.:			574 904