# B.PROTHERM 420 KBRUH with removable heating element

Dimensions

Length:				692 mm

Width:			445 mm

Height:			470 mm

Model

The food transport container is manufactured with the thermoforming process and consists of high-quality, non-hazardous, food-resistant polypropylene.

The double-walled synthetic container and the door are filled with PUR foam for thermal insulation. Inner and outer containers are welded to be leak-free. The synthetic surface is smooth and non-porous.

Stainless-steel carrying handles with ergonomically shaped synthetic carrying handles are located on the long sides of the body recessed in depressions. The stainless-steel carrying handles enable wear-resistant use, even with heavy loading.

The container is equipped with a hinged door that can be swivelled by 270° and removed if necessary. The patented door hinge ensures that the door can only be removed in a predefined position. This prevents accidental removal of the door and the related risk of injury.

The door is locked with a stainless-steel bent-clamp closure on the long side of the container which engages in the door.

A device for holding marking cards is integrated on the door to identify the food in the container.

Two stacking skids are screwed onto the underside of the container, and the top is provided with for screwed-on stacking corners. All B.PROTHERM containers can be stacked with one another.

The container is dishwasher-safe without the door or with the heating element removed up to a maximum temperature of +90 °C.

All wearing parts (skids, stacking corners, handles, door hinges and door lock) can easily be replaced if service is required. All countersunk screws are covered with synthetic plugs for hygienic reasons.

The BPT 420 KBRUH is loaded from the front. The interior body of the container is equipped with 11 pair of moulded support ledges with a spacing of 31 mm for holding Gastronorm containers.

The convection heating module is secured with two quick-release fasteners (LOXX) on the inside of the hinged door and can therefore be removed without tools.

The electrical connection with a mains plug coupling, which can be locked protected against splashed water with a special rotary lock, is located in the upper area of the door.

The digital control with an LED display is located next to this.

The temperature inside the container can be adjusted down to the degree from +40 °C to +85 °C for keeping food hot in an especially gentle and appetising way.

A temperature of approx. +90 °C is reached in Full-Power mode. The actual temperature inside the container is indicated on the LED display.

The heater is equipped with integrated overheating protection to prevent the food being cooked further.

The mains connection cable is supplied loose with the container.

### Accessories/Options

- Various transport trolleys and dollies (see full price list)

- Eutectic plate -3 °C (Art. No. 568136)

- Eutectic plate -12 °C (Art. No. 573332)

- Slide-in frame for subdivision of GN containers with a max. depth of 150 mm

(Art. No. 564352)

- GN support bars for variable insertion of GN containers of the size GN 1/4, GN 1/6, GN 1/9 in the slide-in frame (ST 3 Art. No. 550650, ST 5 Art. No. 550651)

- Menu cards incl. accessories (see full price list)

Technical data

Material:	Polypropylene (PP), 	polyamide (PA),	stainless steel

Temperature

resistance:	-25 °C to +100 °C

Insulating material:	PUR foam - CFC-free

Ledges:	11 pair of support ledges, spacing: 31 mm

Capacity:	2 x GN 1/1-150

Heat-up time:	A temperature of approx. 75 °C is reached in 45 min. in the Full-Power mode in an empty container

Weight:	14.63 kg

Connected load:	220-240 V AC/50-60 Hz

200 Watt

Emissions:	The workplace-specific noise level of the unit is less than 
70 dB(A)

Special features

- IP X4 protection against splashed water

- Heating element removable without tools

- Dishwasher safe up to +90 °C without door or with heating element removed

- Recyclable

- Temperature control down to the degree from +40 °C

to +85 °C

- Temperature display

- Heater with integrated overheating protection

- Patented door hinge (can only be removed in predefined position)

- Handle at upper left in door for easy movement and optimal handling of the container

Make

Manufacturer:			B.PRO

Model:	B.PROTHERM

BPT 420 KBRUH

Order No.			575 600