# B.PROTHERM 620 KBUH with removable heating element

Dimensions

Length:				692 mm

Width:			440 mm

Height:			660 mm

Model

The food transport container is manufactured with the thermoforming process and consists of high-quality, non-hazardous, food-resistant polypropylene.

The double-walled synthetic container and the door are filled with PUR foam for thermal insulation. Inner and outer containers are welded to be leak-free. The synthetic surface is smooth and non-porous.

Stainless-steel carrying handles with ergonomically shaped synthetic carrying handles are located on the long sides of the body recessed in depressions. The stainless-steel carrying handles enable wear-resistant use, even with heavy loading.

The container is equipped with a hinged door that can be swivelled by 270° and removed if necessary. The patented door hinge ensures that the door can only be removed in a predefined position. This prevents accidental removal of the door and the related risk of injury.

The door is locked with a stainless-steel bent-clamp closure on the long side of the container which engages in the door.

A device for holding marking cards is integrated on the door to identify the food in the container.

Two stacking skids are screwed onto the underside of the container, and the top is provided with for screwed-on stacking corners. All B.PROTHERM containers can be stacked with one another.

The container is dishwasher-safe without the door or with the heating element removed up to a maximum temperature of +90 °C.

All wearing parts (skids, stacking corners, handles, door hinges and door lock) can easily be replaced if service is required. All countersunk screws are covered with synthetic plugs for hygienic reasons.

The BPT 620 KBUH is loaded from the front. The interior body of the container is equipped with 12 pairs of moulded support ledges with a spacing of 39 mm for holding Gastronorm containers.

The convection heating module is secured with two quick-release fasteners (LOXX) on the inside of the hinged door and can therefore be removed without tools.

The electrical connection with a mains plug coupling, which can be locked protected against splashed water with a special rotary lock, is located in the upper area of the door. The operating mode indicator is located next to this. It lights up as soon as the mains voltage is connected to the unit.

The temperature inside the container is limited to 
+90 °C. The heater is equipped with integrated overheating protection to prevent the food being cooked further.

The mains connection cable is supplied loose with the container.

### Accessories/Options

- See full price list for various transport trolleys and dollies

- Eutectic plate -3 °C (order No. 568136)

- Eutectic plate -12 °C (order No. 573332)

- Slide-in frame for subdivision of GN containers with a max. depth of 150 mm

(order No. 564 352)

- GN support bars for variable insertion of GN containers of the size GN 1/4, GN 1/6, GN 1/9 in the slide-in frame (ST 3 order No. 550 650, ST 5 order No. 550 651)

- See full price list for menu cards incl. accessories

Technical data

Material:	Polypropylene (PP), 	polyamide (PA),	stainless steel

Temperature

Resistance:	-25 °C to +100 °C

Insulating material:	PUR foam - CFC-free

Ledges:	12 pair of support ledges, spacing: 39 mm

Capacity:	2 x GN 1/1-200

Heat-up time:	A temperature of approx. 75 °C is reached in 45 min. in an empty container

Weight:	18.55 kg

Connected load:	220-240 V AC/50 Hz/

200 Watt

Emissions:	The workplace-specific

noise level of the unit is

less than 70 dB(A).

Special features

- IP X4 protection against splashed water

- Heating element removable without tools

- Dishwasher safe up to +90 °C without door or with heating element removed

- Recyclable

- Operating mode indicator

- Heater with integrated overheating protection

- Patented door hinge (can only be removed in predefined position)

- Recessed handle at upper left in door for easy movement and optimal handling of the container

Make

Manufacturer:			B.PRO

Model:	BPT 620 KBUH

B.PROTHERM

Order No.			575 603