Gastronorm – Container, perforated

GN-P 2/1-100

Dimensions

Exterior dimensions:

Length 		650 mm

Width:		530 mm

Depth:		100 mm

Inside dimensions:

Length:		625 mm

Width:		505 mm

Base radius:		R 30 mm

Corner radius:	R 80 mm

# Design

This Gastronorm container with a perforated base and side is made of AISI 304 stainless steel and is seamlessly deep-drawn. The base and side wall feature a perforated section with holes 4 mm in diameter and 4 mm spacing between holes. The hole pattern designed especially for cooking behaviour ensures shorter cooking times and faster cooling or draining for food. The all-round, highly pronounced stacking collar ensures that containers are stacked or unstacked evenly and also helps prevent jamming. Exceptionally large corner and base radii ensure that food is completely used up while also making cleaning easy. The all-round edge embossment makes the entire edge more stable and more rigid. The GN-P container features drop-shaped embossments on its edges. The functional corners ensure sturdy corner sections and extreme shock resistance and allow food to be tipped out in the required quantity.

Technical data

Material:	AISI 304 stainless steel

Standards:	DIN EN 631

Capacity:	up to 20 mm below edge: 	23.3 litres

up to 3 mm below edge:

28.9 litres

Weight:	2,55 kg

# Special features

- Large corner radii (side wall to side wall)

- Increased base radii (base to side wall)

- Container edge with functional corner

- Base and sides perforated

Make:

Manufacturer:	B.PRO

Model:	GN-P 2/1-100

Order No.:		  1 565 791