# Flat-belt conveyor GSPV

Dimensions

Length:	3,000 – 15,000 mm

Width:         	500 mm

Height:          	900 mm

Model

The belt conveyor, made of chrome-nickel-steel 18/10, is a self-supporting, stable and torsion-free unit. The conveyor table is supported approximately every 2,500 mm in the form of height-adjustable square tube feet 40/40 mm. The thickness of the conveyor material is 1.5 mm.

The conveyor sections are joined by a screw joint/insertion seam.

The belt is permanently cleaned by a dirt stripper with a collection container which can be emptied. The dirt stripper can be removed for cleaning together with the collection container.

The unit is driven with a drum motor built into the conveyor body. The deflection roller is made of CNS 18/10 and located in a tensioning station. The belt guide in the underframe is freely accessible and is led through ball-bearing synthetic castors.

The conveyor belt is white, of abrasion- and food-resistant quality and welded together in an endless loop. The underside of the belt has antistatic coating.

The sensor limit switch is located at the end of the conveyor together with the on/off button and the main switch. An emergency stop button is mounted at both the beginning and end of the conveyor.

The controller, including the motor-protection switch, is integrated in a switch cabinet made of CNS 18/10. Wiring is executed according to VDE guidelines.

It is mandatory to fit socket outlets with an RCCB.

The drive can be regulated continuously from 2.5-12 m/min.

### Accessories/options

- 230V Schuko socket outlet

- 230 V CEE socket outlet

- 400 V CEE socket outlet

- Cable duct on both sides

- Additional ON/OFF button

- Foot switch with isolating transformer (ON/OFF)

- Bumper rail profile on both sides

- Swing-out table for patient cards (max. load: 5 kg)

- Folding table

Technical data

Material:		CNS 18/10

Support:		every 2,500 mm

Max. permissible load per m:	120 N/m

Drive regulation:		2.5 – 12 m/min

Connected load:		230 V/ 400 V/ 			3/N/PE

Connected load

Drum motor:		230 V/ 50 Hz/     		0.16 kW

Standards:	Protection type IP 
	X5 according to 
	DIN 60529; CE 
	conformity; 
	Socket outlets IP 
	X4

Emissions:	The workplace-specific noise level of the unit is less than 70 dB(A)

### Special features

- Maintenance-free drum motor

Make

Manufacturer:			B.PRO

Model:	GSPV

Order No.:	383736