# Cooling station for RWR 161 KS

# KS-UK RWR 161

Dimensions

Length:				approx.    780 mm

Width:				approx. 1,095 mm

Height:				approx. 1,940 mm

###### Application

Cooling station for RWR 161 KS, RWR 161 and RWRR 161 shelf trolleys.

For maintaining the cold temperature of food supplied in the shelf trolley for portioning on the food distribution conveyor.

The cooling station is connected to a customer-supplied refrigeration system.

###### Model

Design

The cooling station is a fixed-position floor unit and stands on six feet made of stainless steel, which can be adjusted to compensate for uneven floors.

The cooling station is permanently anchored to the floor with one fixing disk for each foot.  The fixing disks are included in the scope of delivery.

The unit body is made of torsion-resistant stainless steel. The surface is micro-polished. The cooling station consists of two side walls, one rear wall and a top. The body is double-walled and insulated with polyurethane foam panels.

An air intake duct is located in the middle of the lower area of the cooling station. It can be folded upward for cleaning.

Additional guide rails are located on the left and right sides and simplify handling when rolling in the shelf trolley.

The intake duct in the middle and the side guide rails (roll-in aids) form a unit.

The main cooling system, including fans, air guidance and condensation-water collecting unit, is integrated in the rear wall of the body. There is also a revision panel for servicing in the lower area.

Two additional air intake ducts are located at the bottom on the left and right sides. These form the second cold-air system together with the fans, the second refrigeration package integrated in the top and the air exhaust opening on the front. This second cold-air system makes use of a cold-air curtain at the front, which prevents an air exchange of the cold air in the interior with the warm room air.

The two side air intake ducts can be removed without tools for cleaning.

The central On/Off switch and the digital temperature controller are integrated in the body top in an easily accessible, highly visible location on the operator side.

The mains connection cable with a Schuko plug and the connections for the refrigerant and the condensation water can be guided out of the unit on the the left, right or centre.

##### Active convection cooling

Cooling is controlled by the digital temperature controller.

The temperature can be regulated down to the degree from +4°C to +12°C (at an ambient temperature of +32°C).

The powerful convection fan circulates the cold air in the unit, and therefore also in the shelf trolley. The cold air is exhausted uniformly on all levels via the air baffle on the rear wall.

The cold air glides over the shelf trolley and is then guided through the intake duct again and then through the refrigeration package in the rear wall of the body.

Part of the air is fed to the cold-air curtain at the front.

The intake air is drawn through removable and replaceable expanded wire mesh filters. This prevents any dirt and impurities, particularly from the kitchen floor, from being drawn into the interior of the cooling station.

### Accessories/options

- Refrigeration connection, condensation water drain and mains connection cable can be positioned to the left or right or in the middle.

- XJ485 data interface, for integration into an external control centre and data reporting

- Castors in place of adjustable feet, 6 steering castors, 2 of which have brakes, castor diameter 75 mm Enables more flexible use on the food distribution conveyor and facilitates the cleaning process. Only possible in conjunction with cooling brine as a refrigerant and quick-release couplings.

- Condensation water evaporator, electrically heated

- Refrigerant either R134a, R404A, R449A,

R513A or cooling brine (water glycol)

- Quick-release couplings: Quick-release couplings allow the refrigerant lines to be connected to the on-site refrigeration circuit quickly and easily. Only in conjunction with refrigerants (glycol, cooling brine) – not available in conjunction with classic refrigerants.

- Double-walled, insulated door for using the cooling station like a roll-in refrigerator (the door hinging is always located on the opposite side to the refrigeration connection)

- Door hinging left or right

Technical data

Material:		Stainless steel

Insulating material:	Polyurethane (PU)

Weight:		Approx. 255 kg

Connected load:		220-240 V AC/ 50 Hz/ 0.2 kW

Refrigerating capacity

to be installed: 			2.8 kW (at t0 = -10°C)

Emissions:	The workplace-specific noise 
level of the unit is less than 70 dB(A).

Special features

- Active convection cooling

- Connection to a customer-supplied refrigeration system

- Protection type IP X5

Make

Manufacturer:			BLANCO

Model:				KS-UK RWR 161

Order no.			389047