# Shelf Trolley RWR 160-20

Dimensions

Length:				  461 mm

Width:			  614 mm

Height:			1801 mm

Inside dimensions:

Spacing between support rails:	    73 mm

Design

The shelf trolley consists of stainless steel, AISI 304. U-profile guide rails are welded between the sturdy 25 x 25 mm square tube. These are supplied with a tip safety and push-through protection on both sides for GN 1/1, 2/3, 1/2,1/3 and 2/8.

A 20 x 10 mm square tube welded in at mid-height offset on both the front and back of the shelf trolley provides additional stability.

The shelf trolley is mobile on corrosion-resistant synthetic castors pursuant to DIN 18867, Part 8

(4 steering castors, 2 of which have brakes, castor diameter 125 mm). Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories/options

- CNS panelling, 2 or 3-sided

- Transport locking bar on both sides

- Push handle short side

Technical data

Material:			Stainless steel

Polyethylene (PE)

Material thickness of the 
square tube:	1.25 mm

Material thickness of the

support rails:			1.25 mm

Weight:		19 kg

Load-bearing capacity
per trolley:		200 kg

Max. load-bearing

capacity per support pair: 	30 kg

Number of support pairs:	20 pairs GN 1/1 or their subdivision

Capacity:		9 x GN 1/1-150 without lid

### Special features

- U-profile guide rails with push-through protection

Make

Manufacturer:		B.PRO

Model:	RWR 160-20

Order No.:		572 303