# RWR 162 Shelf Trolley

Dimensions

Length:				  820 mm

Width:			  613 mm

Height:			1645 mm

Interior dimensions:

Space between support rails:	    73 mm

Design

The shelf trolley consists of CNS 18/10 (AISI 304), Material No. 1.4301. U-profile guide rails are welded between the stable 25 x 25 mm square tubes (curved without a recess). These are supplied with a tip safety as well as push-through protection on both sides for GN 2/1, 1/3 and 2/8. The shelf trolley is two-part.

A 20 x 10 mm square tube welded in elevated at mid-height on both the front and back of the shelf trolley provides additional stability.

The shelf trolley can be moved on galvanized steel

castors (4 steering castors, 2 of which have locking brakes, castor diameter 125 mm). Solid synthetic (polyamide) wall guards at all four corners protect against damage.

Accessories and options

- Dust protection hood made of transparent polyethylene (Order No. 568 773)

- Insulating hood made of polyethylene, aluminized on both sides (Order No. 568 776)

- Trolley top made of CNS

- CNS paneling, 2 or 3-sided

- Transport circlip, double-sided

- Push handle

Technical data

Material:			Chrome-nickel-steel 18/10 (AISI 304),

Polyamide (PA)

Material thickness of the

square tube:			1.5 mm

Material thickness of the

support rails:			1.2 mm

Weight:		29 kg

Load-bearing capacity

per trolley:		200 kg

Max. load-bearing

capacity per support pair: 	30 kg

Number of support

pairs:			36 pairs GN 1/1 or their subdivision

Capacity:		17 x GN 1/1-150 without lid

### Special features

- U-profile guide rails with tip safety and push-through protection on both sides

- 2-part model

Make

Manufacturer:		B.PRO

Model:                               RWR 162

Order No.:		572 941