# RWR 3-A KS shelf trolley

Dimensions

Length:				1,171 mm

Width:			   818 mm

Height:			1,206 mm

Spacing between support rails:	     82 mm

Model

The shelf trolley consists of stainless steel. U-profile guide rails are welded between the sturdy 
25 x 25 mm square tube. These are supplied with a tip safety as well as push-through protection.

The 7 support pairs each of the three-part trolley are used for sliding in GN containers, GN trays or GN grates. In addition, 3 GN containers of the size GN 1/1 can be hooked in from above. The trolley is provided with an attachment mounted at a 10° angle, which can hold 3 GN containers of the size GN 1/1 (hooked in from above).

To prevent cold loss, the shelf trolley has a one-piece stainless steel bottom plate.

The shelf trolley is mobile on corrosion-resistant synthetic castors pursuant to DIN 18867, Part 8

(4 steering castors, 2 of which have brakes, castor diameter 125 mm). Synthetic (polyamide) wall guards at all four corners protect against damage.

Technical data

Material:			Stainless steel, 			polyamide (PA)

Weight:		38 kg

Load-bearing capacity 
per trolley:		200 kg

Number of support 
pairs:		3 x 7 pieces

Capacity:		27 x GN 1/1 or

54 x GN 1/2

### Special features

- U-profile guide rails with push-through protection

- 3x GN containers of the size GN 1/1 can be hooked in from above (ideal for portioning on food distribution conveyor)

- Attachment mounted at a 10° angle for holding 3 GN containers of the size GN 1/1

Make

Manufacturer:			B.PRO

Type:	RWR 3-A KS

Order No.			573386