# MCS 10x6-3 mobile clearing station

Dimensions

Length:				1100 mm

Width:			  700 mm

Height:			  950 mm

Inside dimensions:

Height between shelves:		  275 mm

Shelves:		 1000 x 600 mm

Model

Mobile clearing station MCS 10x6-3 based on a 10x6 serving trolley in a basic design.

The mobile clearing station is made of stainless steel, AISI 304. There are three welded-in, deep-drawn shelves between a sturdy round tube 25 mm in diameter. The shelf surfaces are micro-polished. The round tubes on the short sides are used as push handles. A 35-mm turned-down, all-round profile edge on the shelves, the roller-burnished shelf rim and single-piece welded seams between the tube frame and shelves all enhance the clearing station’s stability.

All shelves are lined noise-insulating anti-drumming mat.

The top shelf contains a cut-out in which two Gastronorm containers with GN-B 1/3-200 U-handles are positioned. The clearing station features 4 gusset plates each on the upper two shelves for additional stability.

The clearing station is mobile on corrosion-resistant synthetic castors as per DIN 18867, Part 8 (4 steering castors, 2 of which have brakes, castor diameter 125 mm). The castors are inserted into the tube frame. Synthetic (polyethylene) wall guards at all four corners protect against damage.

Accessories

- Drip tray with handles, for glass racks, 
500 x 500 mm, made of stainless steel, placed on the top shelf (order no.: 574 779)

- Stainless-steel storage shelf, fitted at standard height or children's height (850 or 700 mm) without tools

(order no. 574 780)

- Hanging frame for GN containers, made of stainless steel

(order no. 574 781)

Offers space for up to 3 Gastronorm containers GN 1/6

Panelling and privacy shield:

- Thin-sheet panelling on 3 sides, electrolytically galvanised on both sides, powder-coated in various colours; for hanging on the clearing station, order no.: 375455

- Thin-sheet privacy shield, electrolytically galvanised on both sides, powder-coated in various colours; for easy mounting on tube clamps, order no.: 382662

Technical data

Material:			Stainless steel,

polyamide (PA)

Weight:		Approx. 23 kg

Load-bearing capacity
per trolley:		150 kg

Load-bearing capacity
per shelf:		80 kg

Number of shelves:		3

Max. capacity:		45 people

### Special features

- All-round profile edge on the shelves to reinforce the stability of the clearing station and prevent items on the trolley surface sliding off the sides.

- Roller-burnished shelf rim: curled edges ensure improved stability and protect staff against injury.

Make

Manufacturer: 	                 B.PRO

Model:                                MCS 10x6-3

Order no.                           575 520